## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Materials & Structures(CE402) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Stress, Strain and Young’s modulus for axial compression. |  |
| b) | What is Shear Stress, Shear Strain and Modulus of rigidity? |  |
| c) | Explain stress-strain relationship for mild steel subjected to axial tension. |  |
| d) | A material has a Young’s modulus 1.25 x 105 N/mm2 and a Poison’s ratio of 0.25. Calculate the Modulus of rigidity and the bulk modulus. |  |
| e) | Explain how maximum stress intensity due to a suddenly applied load is twice the stress intensity produced by the load of same magnitude applied gradually. |  |
| f) | Locate the center of gravity of inverted right circular cone and inverted hemisphere. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A bar consisting of three length 180mm,260mm and 160mm with diameters 30mm , 20mm  and 40 mm  respectively. Find the stresses in the three parts of the bar and the total extension produced if the bar is subjected to axial pull of 40 KN. E = 2 x 105 N/mm2. |  |
| b) | A solid steel bar 500 mm long and 70 mm diameter is placed inside in aluminium tube having 70 mm inner diameter and 100 mm outside diameter. The composite bar is subjected to axial load of 600 KN. Find the stress in each material and load shared by each material. ES = 2.2 x 105 N/mm2 Ea = 0.7 x 105 N/mm2 |  |
| c) | A steel rod is 25m long at a temperature of 20C. Find the free expansion of the rod when the temperature is raised to 65C. Also, find the temperature stress produced when i) expansion of the rod is prevented ii) when the rod is allowed to expand by 6 mm.  = 12 x 10-6 perC , E = 2 x 105 N/mm2 |  |
| Q.No.3. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Find the centroid of the lamina shown in fig.1. |  |
|  | ### OR |  |
| a) | Find the center of gravity of the composite solid shown in fig.2. |  |
| b) | Find the moment of Inertia of the section shown in fig.3 about centroidal X–X axis. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cantilever AB fixed at A ad free at B is with point load of 5 KN at free end and uniformly distributed load of 2 KN/m over the entire span of 5m. Draw S.F.D. and B.M.D. Refer fig.4. |  |
| b) | A simply supported beam AB is 8m in span and carries 3 point loads as shown in fig.5. Draw S.F.D. and B.M.D. | ……2/- |
|  | -2- |  |
| c) | A Beam ABCD with supports at B and C at distance 3m apart has overhangs of AB and CD of length 1m each on either side of supports. Points loads 10 KN each are at A and D. Draw the S.F.D. and B.M.D. Refer fig.6. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is Bending Equation? What are the parameters involved in bending equation? State the consumption made in theory of pure bending. |  |
| b) | A beam 20mm x 20mm in section, 1m long simply supported fails when central load 640N is applied to it. What u.d.l. will break a cantilever of the same material 50mm x 100mm in section and 2 m long? |  |
| c) | A ‘tee’ section of flange 120mm x 50mm and web thickness 25mm overall depth 240 mm is subjected to sagging moment. Draw the bending stress distribution diagram if stress in maximum compressive stress is 45N/mm2. Find the sagging moment. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A load of 100N falls by gravity a vertical distance of 300mm when it is suddenly stopped by a collar at the end of a vertical rod of length 6m and diameter 20mm. The top of the bar is rigidly fixed to the ceiling. Calculate the maximum stress and the strain induced in the bar. Take E = 1.96 x 105 N/mm2. |  |
| b) | A steel beam of I section, top and bottom flange 250mm x 25mm, web thickness 15mm thick and overall depth 600mm is subjected to a shear force of 500 KN. Determine the shear stress distribution for the beam section. |  |
| c) | A beam of circular section is 150mm diameter. If the beam be subjected to a maximum shear force of 7000N, find the average shear stress and maximum shear stress. Draw the shear stress distribution diagram across the section. |  |