## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Automobile Engineering

Subject: Automobile Engines-II(4061)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain with sketches various piston head shapes and write the functions of cup and hollow in piston head. |  |
| b) | With sketch explain any two methods of supercharging. |  |
| c) | Explain the causes of low lubricating oil pressure. |  |
| d) | What is sludge? How does it form? How can it be prevented? |  |
| e) | Draw a neat diagram of Saybolt Viscometer and label all its parts. |  |
| f) | Explain in brief the Morse Test. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between LHV and HHV of fuels. Describe a method to find HHV of a fuel. |  |
| b) | Explain the function of a governor. Sketch a Mechanical governor and explain its working. |  |
| c) | Describe with a clear sketch the Idling circuit in Solex-Mikuni double venturi carburettor. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the conventional Battery coil ignition system for a 6-cylinder petrol engine. |  |
| b) | Describe with the help of a sketch, the centrifugal advance mechanism, and why is this advance needed. |  |
| c) | Describe with the help of a neatly drawn spark-plug, its construction & working. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe cylinder blocks of Aluminium and sleveless aluminium, ring-groom fortification. |  |
| b) | Explain the electric car, its merits and demerits. Also explain the parallel system hybrid car. |  |
| c) | Compare between Supercharging and Turbocharging. Also, explain with a neat sketch the operation of “waste-gate” in turbocharging. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the exhaust emission control by reducing combustion chamber surface area and by using stratified charge. |  |
| b) | Explain with sketch three-way catalytic converter. |  |
| c) | With the help of a neat sketch, explain the working of a gear type lubricating oil pump. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure for calculating IHP by an electronic indicator. |  |
| b) | Describe the principle of working of a NDIR analyser. |  |
|  |  | ….2/- |
|  | -2- |  |
| c) | A single cylinder engine is working on a 4-stroke cycle. It was tested with a Rope-Brake and the following readings were noted: Brake wheel diameter = 600mm Rope diameter = 26mm Spring Balance reading S = 38 N Dead weight on the brake W = 190N Find the value of Torque and BP. |  |

***********