## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Computer Engg.

Subject: Microprocessor & Its Applications(CO401) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the functions of the following pins/signals in 8086: i) RD ii) AD0−AD15 iii) RESET |  |
| b) | State any three features of Pentium Processor. |  |
| c) | Explain the role of stack pointer register in 8086. |  |
| d) | Explain the following instructions of 8086 with an example: i) DEC ii) LODSB/W. |  |
| e) | Explain Based and Relative-Based addressing mode with an example. |  |
| f) | Discuss the role of control flags in flag register of 8086. |  |
| g) | Discuss in brief the BIU section of 8086. |  |
| h) | Explain Recursive procedure with a diagram. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the functional block diagram of 8085. |  |
| b) | Explain the role of Execution unit in 8086. |  |
| c) | Discuss on the functions performed by the Microprocessor. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List and explain the processor control Instructions. |  |
| b) | Given: i) AX=1211H ii) BX = 4021H iii) CL=03H Explain the result of each of the following instructions and state the flags affected: i) SUB AX, BX ii) RCL BH, CL iii) CMP BX, AX |  |
| c) | Explain If-Then and If-Then-Else structure with an example. Write an Assembly Language code to add 2 16-bit Hexadecimal values. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss stack and explain how stack operations are carried out in 8086. |  |
| b) | Explain how CALL and RET instructions are used for implementing a procedure in 8086. |  |
| c) | Explain the following Assembler Directives. State an example for each and explain. i) DUP ii) DB iii) SEGMENT |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain write machine cycle with the help of a neat waveform diagram. |  |
| b) | Explain 8086 in maximum mode. Explain the function of the pins used in maximum mode. |  |
| c) | Draw and explain the Interrupt Pointer table in 8086. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with diagram interfacing of 8086 with Traffic light controller. |  |
| b) | Explain with diagram interfacing of 8086 with Analog to Digital converter. |  |
| c) | Explain with diagram interfacing of 8086 with Digital to Analog converter. |  |

***********