## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2015 Examinations

Programme: Electronics Engg./EC/EI

Subject: Analog Communication(EN402) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is modulation? What are the principal types of modulation used in communication system? |  |
| b) | Define modulation index of AM. What is the range of modulation index in AM. |  |
| c) | What is AGC and why is it used in communication system. |  |
| d) | Define FM. Draw waveforms of FM signal. |  |
| e) | Draw the block diagram of FM receiver. |  |
| f) | Define signal to noise ratio and noise figure of a receiver. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain generation of AM using grid modulated class C amplifier. |  |
| b) | i) Explain the need of modulation. ii) Draw the frequency spectrum of AM wave and explain. |  |
| c) | Unmodulated RF carrier power of 10 KW sends a current of 10A rms through an antenna. On amplitude modulation by another sinusoidal voltage, the antenna current increase to 11.6 A, calculate (i) the modulation index (ii) carrier power after modulation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a block diagram explain filter method for suppressing a sideband. |  |
| b) | Explain with the help of block diagram low level AM transmitter. |  |
| c) | Explain with the help of block diagram TRF receiver. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare FM and AM systems. |  |
| b) | Differentiate between narrowband and wideband FM. |  |
| c) | Explain pre-emphasis and de-emphasis in FM system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Draw the block diagram of stereophonic FM transmitter. ii) Define radiation resistance and polarization of an antenna. |  |
| b) | Explain with a neat circuit diagram balanced slope detector for demodulations of FM signal. |  |
| c) | Draw block diagram of communication system. Write a note on various frequency bands used for communication. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Extraterrestrial noise |  |
| b) | Parabolic reflector antenna |  |
| c) | Any two internal noise that get generated in communication system |  |
| d) | Co-axial cable |  |

***********