## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Structural Drawing(4341)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three of the following: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Wave bending moment |  |
| b) | Ship as a hull girder |  |
| c) | Bouyancy Curve |  |
| d) | Sheer Strake |  |
| Q.No.2. | Sketch the midship section of a bulk carrier showing all structural details. Name the following members: i)Scallop ii)Strut iii)Sheer Strake iv)Hatch Coaming v)Keel Plate. | (12) |
| Q.No.3. | A box shaped Barge of 60m length and light displacement of 480 tonnes is divided by transverse bulkheads into four equal holds. Each hold is loaded with uniformly distributed cargo as follows: | (12) |
|  | Hold No. 1; Hold No. 2; Hold No. 3; Hold No. 4. 200 T; 260 T; 200 T; 220 T. |  |
|  | Construct load, shear force and bending moment diagram. Find the maximum bending moment. |  |
| Q.No.4. | Answer the following: |  |
| a) | Explain how shell expansion is obtained from Body plan. | (6) |
| b) | Explain the use of shell expansion during the construction of the vessel. | (6) |
| Q.No.5. | Determine the moment of Inertia about XX axis and the section modules of an angle sectioned longitudinal of dimension 100mm x 100mm x 8mm along with the plate of 12mm to which it is welded with the spacing being 500mm as shown below: (see fig. on p.g.2) | (12) |
| Q.No.6. | Answer the following: |  |
| a) | Why midship section in way of hatch openings is considered for hull girder section modulus calculation? | (6) |
| b) | What is hogging and sagging? | (6) |
| Q.No.7 | Explain in detail how weight curve, bouyancy curve, load curve, S.F diagram and B.M. diagrams are obtained for loaded ship structure. | (12) |