## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Machine Technology(4217)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Inversion of a kinematic chain. Explain any one inversion of a slider crank mechanism. |  |
| b) | How are gears classified? State the terms used in the nomenclature of Gears. |  |
| c) | Distinguish between Fire tube and water tube boilers. |  |
| d) | Explain “Law of Belting” with reference to flat belt drive. |  |
| e) | Why is a condenser used in a steam power plant? |  |
| f) | Explain operating principle of Rotary compressor. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What is meant by cross – belt drive? Find the length of the belt in a cross – belt drive. |  |
| b) | Explain Whitworth’s Quick return motion mechanism with the help of a neat sketch. |  |
| c) | Two parallel shafts 6 meter apart are provided with 300mm and 400mm diameter pulleys and are connected by means of cross belt. The direction of rotation of the follower pulley is to be reversed by changing over to the open belt drive. How much length of belt has to be reduced? |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | State and explain the classification of Kinematic pairs. Sketch the various pairs used in Engineering applications. |  |
| b) | Explain with neat schematic sketch Lancashire Boiler. Name its important parts. |  |
| c) | What are couplings? Give their applications. Differentiate between Rigid couplings and flexible couplings. |  |
| Q.No.4. | Write brief note on any three of the following: | 3 x 4 = 12 |
| a) | Classification of steam turbines |  |
| b) | Evaporative condenser |  |
| c) | Reciprocating Air compressor |  |
| d) | Velocity compounding in steam turbines |  |
| e) | Economiser and Air Preheater |  |
| Q.No.5. | Answer any three of the following: | 3 x 4 = 12 |
| a) | Differentiate between Jet Condensers and surface condensers. (at least four Points) |  |
| b) | Why steam turbines need compounding? Explain with neat diagrammatic sketch. |  |
| c) | Differentiate between centrifugal pump and Rotary pump. |  |
| d) | Explain Double slider crank mechanism and its Inversions. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any three of the following: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Vacuum pump |  |
| b) | Water level indicator and Bourden tube Pressure gauge in Boilers |  |
| c) | Air and Vacuum chamber in pumps |  |
| d) | Epicyclic gear train |  |
| e) | Principle of operation of simple impulse turbine |  |

***********