## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics Engineering/EI/E/EC/COMP/MED/E&EE

Subject: Digital Electronics-I(4134)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Convert the following to binary i)2710 ii)358 iii)FA616. |  |
| b) | Perform the following addition using 2’s complement arithmetic: 9410 +(-37)10 |  |
| c) | Using Boolean laws, prove that (A+B)(A+B)(A+C)=AC |  |
| d) | Explain the race condition in JK flip flops. |  |
| e) | - What is the modulus of a 6- bit counter? - How many flip flops are required to construct a 6- bit counter? - Determine the largest decimal number that can be represented by a 6- bit counter. |  |
| f) | Define the following characteristics of digital logic families: - Fan-out - Power dissipation - Noise margin. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Draw the simplest NAND –NAND circuit for the following Y (A,B,C,D)= ∑ m (2,3,8,9,10,11) + d(12,13,14,15) |  |
| b) | Simplify and implement using logic gates i) A(B+C)(A+B+C)ABC ii) ABC+ABC+ABC |  |
| c) | Implement the following gates using NOR gates only i) OR gate ii) AND gate iii) EX-OR gate. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Draw the logic diagram of a D flip flop using NAND gates. Explain why the flip-flop is called a latch. |  |
| b) | Construct a T flip flop form a basic RS FF. Explain the response of the T flip flop for a low input and a high input. |  |
| c) | Draw the logic diagram of a clocked JK FF. Write its truth table and explain its working. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Draw the logic diagram of a 3- bit PISO register using D flip-flops. Explain how data is written to the register and read form the register. |  |
| b) | Draw the logic diagram of a 3- bit asynchronous up counter. Draw the output waveforms and write the different states the counter goes through. |  |
| c) | With the help of a logic diagram explain the operation of a mod 4 synchronous down counter. Draw the output waveforms. |  |

………2/-

-2-

| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw the circuit and explain the working of a CMOS 2- input NOR gate. |  |
| b) | Draw the basic circuit of an ECL 2-input NOR gate. Explain how it functions as a NOR gate. |  |
| c) | Compare the standard TTL, ECL and CMOS family w.r.t the following characteristics: i)Power dissipation ii)Propagation delay. |  |
| Q.No.6. | Write notes on any two: (Draw diagrams wherever necessary) | 2 x 6 = 12 |
| a) | i) ASCII code ii) BCD code |  |
| b) | Open collector TTL gate |  |
| c) | 3-bit Ring counter using D flip flops |  |

***********