## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg./P/TD

Subject: Metrology & Quality Control(4047)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the scope of metrology? |  |
| b) | Name basic standards of linear measurement. Explain “Line Standards.” |  |
| c) | Name types of Fits. Explain “Transition Fit”. |  |
| d) | Explain with the help of a sketch how straightness is measured using spirit level. |  |
| e) | Explain with a neat sketch how you measure effective diameter of screw thread using “Thread – Micrometer method”. |  |
| f) | State and explain any three important objectives of “Quality Control”. |  |
| g) | What is Quality Mindedness? Explain how you create quality mindedness in an organization. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the following: (i) Controllable errors (ii) Random errors. |  |
| b) | Name various instruments used for angular measurement. An angle of value 102 8 42 is to be measured with the help of following standard angle gauges and a square block. (1, 3 9, 27, 41), (1, 3, 9, 27), (3, 6, 18, 30) Show the arrangement of angle gauges selecting minimum number of gauges. |  |
| c) | State and explain with neat sketches “Taylor’s principle” as applied to design of limits gauges. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Name various types of comparators. State 2 advantages and 2 disadvantages of pneumatic comparators. |  |
| b) | With the help of a neat sketch explain the construction and operation of a stylus type surface roughness measuring instrument. |  |
| c) | With the help of a neat sketch explain the use of “Parkinson’s Gear Tester”. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the meaning of “Quality of Design”. State the factors controlling Quality of Design. |  |
| b) | Explain with suitable example, “Cost of Quality” and “Value of Quality.” Explain with the help of a sketch how to obtain balance between the two. |  |
| c) | Explain the following functions: (i) Inspection (ii) Quality Control. |  |

………2/-

-2-

| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What is “Vender Quality Rating”? State any three uses of vender Quality Rating. |  |
| b) | Write short notes on: (i) Process capability (ii) Analysis of field complaints. |  |
| c) | With the help of neat sketch of O.C curve, Explain: - Producer’s risk (ii) Consumer’s risk (iii) LTPD (iv) Indifferent Quality Region. |  |
| Q.No.6. | Answer any two: | 2 x 6 = 12 |
| a) | X and R chart are to be maintained for a certain dimension of a component. Subgroup size is 5. After 20 subgroups it is found that X = 500 and R=2.00mm. Complete the central lines and the control limits for X and R charts. Given: N; A2; D3; D4. 2; 1.88; 0; 3.27. 3; 1.02; 0; 2.57. 4; 0.73; 0; 2.28. 5; 0.58; 0; 2.11. |  |
| b) | Compare Xand R charts with P chart. |  |
| c) | Write short Notes on any two: (i) Hole based system (ii) Drunken threads (iii) Designating surface roughness on drawings |  |

***********