## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Civil Engg./CT

Subject: Analysis of Structures(4026)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw neat figures of any 3 end conditions of a column & state the effective length of each. |  |
| b) | Calculate the principal stresses at a point in the web of a girder, the bending stress is 60 N/mm2 (T) & shearing stress is 30 N/mm2. |  |
| c) | Determine the Fixed end moments of a fixed beam with point load at 3 m from Right support, of span 5 m. |  |
| d) | Determine slope & deflection of a S.S beam of 5 m span with central point load of 10 KN if flexural rigidity is 2 x 1012 N–mm2. |  |
| e) | Determine the limit of eccentricity for a hollow circular section of 30 cm external diameter & 25 cm internal diameter if it is subjected to a 10 KN force at the outer edge on the horizontal axis. |  |
| f) | State assumptions made in the Euler’s Column Theory. |  |
| g) | A flitched beam made up of steel & timber is such that the steel is sandwiched between timber sections. Determine moment of resistance if each timber section is 60mm wide & 200mm deep & the steel section is 15mm wide & 200mm deep. fs = 100N/mm2 & ftim = 5N/mm2. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | A timber joist 100 mm wide & 150 mm deep is reinforced by fixing a steel plate 100 mm wide & 10 mm thick attached to the bottom of the timber. Find the moment of resistance of the beam if allowable stresses in timber & steel are 7 N/mm2 & 100 N/mm2. Take Es = 16Etim. |  |
| b) | Show that the point of contraflexure for a fixed beam of span l having an udl of w KN/m over the entire span is at a distance of 0.289 l from the centre of the beam on either side. |  |
| c) | An encastie beam of 8 m span is carrying a udl of 5 KN/m on the right half of the beam. Draw shear force & bending moment diagram after determining FEMs. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | A rectangular pier is subjected to a load of 450KN with an eccentricity of 250 mm about both the axes. Find the stresses in all 4 corners of the 1500mm x 1000mm pier. |  |
| b) | Determine using Macauly’s method, the maximum deflection of a cantilever beam subjected to udl over half its span from the free end. |  |
| c) | Using Macauly’s method, derive expression for maximum deflection for a S.S beam with eccentric point load. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A solid steel column and a hollow steel column, both have the same length and same cross-sectional area, and are fixed at both ends. If the internal diameter of the hollow column is 2/3 of its external diameter, find the ratio of the buclling strengths of solid steel column to hollow steel column. |  |
| b) | A compound column consists of 2 channels placed back to back separated by 200 mm. If the length of the column is 5 m, with both ends hinged, find the safe load the column can carry if f.o.s. is 2. Take a = 1/7500 & fy =320N/mm2. For the channel section, A=45.64cm2,Ixx=6362.6cms4 & Iyy=310.844cms4. |  |
| c) | Determine maximum & minimum stress intensities for a column of 60cms diameter carrying a point load of 70 KN at 20cms from centre. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | At a point in a strained material, the resultant intensity of stress across a plane is 84.84 N/mm2 (T), inclined at 45 to its normal. The normal component of the intensity of stress across the plane at right angles is 40 N/mm2(T). Find the position of principal planes & principal stresses across them. Find value of maximum shear also, analytically. |  |
| b) | In a boiler plate, the principal stresses across two perpendicular planes are 100N/mm2 & 50N/mm2, both tensile. Find the intensity of stress, which acting alone, would produce the same maximum strain. (1/m=0.3) |  |
| c) | At a point in a stained material, the normal tensile stresses are 60N/mm2 & 30N/mm2. Determine by Mohr’s circle the resultant intensity of stress on a plane inclined at 40 to the axis of the minor stress. |  |
| Q.No.6. | Answer any one of the following: | (12) |
| a) | Draw BMD & SFD for any one of the figures fig.(1) or (2) using moment–distribution method. |  |
| b) | Draw BMD for portal frame shown in fg.(3). | Cont.. |