## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Food Technology

Subject: Food Engineering-II(4278)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any ten: | 10x1.5=15 |
| --- | --- | --- |
| 1) | List the criteria required for selection of a proper belt for a belt conveyor. |  |
| 2) | Name the conveyor used for transferring filled milk packets to the packing section and conveyor used for transferring milk powder from spray drier to the silos. |  |
| 3) | How is Reynolds’s number useful in Food industry? |  |
| 4) | Name two devices that are used to measure flow rates. |  |
| 5) | Suggest suitable pumps for pumping: (a) Milk (b) Tomato Paste (c) Vitamin premix solution. |  |
| 6) | List three different types of size reduction equipments. |  |
| 7) | How is particle distribution in a ground product measured? |  |
| 8) | State three controlling parameters for the throughput of a Fruit pulper. |  |
| 9) | List three critical parameters for proper mixing of flavour solution with base product in snacks manufacturing. |  |
| 10) | List three factors affecting the centrifugal force in a centrifugal separation. |  |
| 11) | Arrange the following filtrations in order of increasing filter pore size. (a) Micro filtration (b) Ultra filtration (c) Reverse osmosis. |  |
| Q.No.2. | Answer any three: | 3 x 4 = 12 |
| 1) | Draw sketch of a belt conveyor and indicate the parts. State its limitations. |  |
| 2) | Draw sketch of a Pneumatic conveyor and indicate the parts. State its limitations. |  |
| 3) | Explain the working of a water ring vacuum pump. |  |
| 4) | Explain the working of a Bourden’s Pressure gauge. |  |
| Q.No.3. | Answer any three: | 3 x 4 = 12 |
| 1) | Explain the importance of sieve analysis. Name the different International Standards used for expressing mesh size. |  |
| 2) | Explain the working of a sparkler filter and state its applications in food industry. |  |
| 3) | Explain solid-liquid separation by centrifuge. |  |
| 4) | Draw and explain a typical Reverse osmosis system for water treatment. |  |
| Q.No.4. | Answer any three: | 3 x 4 = 12 |
| 1) | Explain the working of a Planetary mixer. |  |
| 2) | State the importance of mixing in food processing. |  |
| 3) | Describe the working of high shear, high speed mixers. |  |
| 4) | Explain the working of dough Kneaders (Sigmoid). |  |

………2/-

-2-

| Q.No.5. | Answer any three: | 3 x 4 = 12 |
| --- | --- | --- |
| 1) | Explain the working of Hammer mill. State its two advantages and two disadvantages. |  |
| 2) | Explain the working of a meat mincer. |  |
| 3) | Explain the working of a Basket press and state its limitations. |  |
| 4) | Explain the system of particle size separation system in a wheat flour mill. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| 1) | Manometer and its uses |  |
| 2) | Augur filler |  |
| 3) | Sorting of Fruits |  |
| 4) | Concher |  |
| 5) | Rotameter |  |

***********