## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg.

Subject: Tool Engineering-I(Cutting Tools)[5033]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why is a nose radius provided on tool tip? |  |
| b) | Why can relief or clearance angles not be zero or negative? |  |
| c) | What is the criteria for machinability based on surface finish? |  |
| d) | State and explain Taylors Tool life equation. |  |
| e) | What causes a built up edge to form? |  |
| f) | What is the composition of Carbon Tool steel? |  |
| g) | How does cutting fluid improve tool life? |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | The following data relate to an orthogonal cutting process:- Chip thickness = 0.62 mm Feed = 0.2 mm/rev Rake Angle = 15 Calculate 1) Cutting ratio 2) Shear angle. |  |
| b) | Explain the mechanism of chip formation. |  |
| c) | Explain chemical wear and electro-chemical wear. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the different factors influencing surface finish during a metal cutting operation. |  |
| b) | A cutting tool, cutting at 25 mts/min, gave a life of 1 hr between regrinds when operating on roughening cuts with mild steel. What will be its probable life when engaged on light finishing cuts? Take n = 0.125 for rough cuts and n = 0.1 for finish cut. |  |
| c) | Explain the principle of Drawing operation. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain with a graph the effect of cutting speed on total cost of a product. |  |
| b) | Explain various methods of operating a broach. |  |
| c) | State the properties and applications of a diamond tool. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the typical composition of H.S.S. Why is cobalt added to H.S.S.? What are the superior qualities of H.S.S. in comparison to cemented carbides? |  |
| b) | Describe the functions of a cutting fluid in a metal cutting operation. |  |
| c) | Explain different type of cutting fluids used in cutting operations. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Bacterial control of cutting fluids |  |
| b) | Crater wear |  |
| c) | Cubic Boron Nitride |  |
| d) | Gear Hobs |  |

***********