## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: Utilisation of Electrical Energy(4111)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How substances can be heated by radiation? State the formula used to find rate of heat transfer by radiation. |  |
| b) | List any six common materials used as heating elements. |  |
| c) | State difference between Arc Welding and Plastic welding. |  |
| d) | With neat diagram explain Butt welding. |  |
| e) | With neat diagram explain Projection welding. |  |
| f) | State any six requirements of ideal traction system. |  |
| g) | Define & state SI units of i)Luminous flux ii)Illuminance. |  |
| h) | Explain difference between specular reflection and diffused reflection of light. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With neat diagram explain working of a Salt Bath Furnace. |  |
| b) | State the methods used to control temperature in an electric furnace employing heating elements. Explain any two methods in detail. |  |
| c) | A resistance oven employing nichrome wire is to be operated from a 220 volt, single-phase a.c. supply. The oven is to be rated at 16 KW. If the temperature of the element is to be limited to 1170c and the average temperature of the charge is 500c, find the diameter and length of the element wire. Assume i) Radiating efficiency = 0.57 ii) Emmissivity (e) = 0.9 iii) Specific resistance of nichrome = 1.09x10-6 -m. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With respect to Arc welding explain importance of: i)Arc stability ii)Arc Blow. |  |
| b) | Briefly explain construction and working of DC welding generator. How current control is achieved here? |  |
| c) | State advantages and disadvantages of DC welding with respect to AC welding. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | State and explain advantages of electric traction. What are its limitations? |  |
| b) | With neat diagram explain shunt transition method for traction motors? Why such transition is necessary? |  |
| c) | Draw and explain a typical speed-time curve for electric traction. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain construction and working of high frequency core less induction furnace, with neat diagram. |  |
| b) | Explain principle of dielectric heating. State and explain its applications. |  |
| c) | A large auditorium has an air circulation of 50,000m3 for 20 hrs. What is the rating of the heater that raises the temperature by 24c? Assume specific heat if air to be 1.34 KJ/m3 c. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With neat diagram explain working of florescent tube. |  |
| b) | For road lighting explain i) Cut off luminaire ii)Semi cut off luminaire. |  |
| c) | Write a short note on lamp fittings. |  |

***********