## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Instrumentation-I(S-4214)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the objectives of measurements? What is the need for control Instrumentation? |  |
| b) | Explain the classification of instruments as: - Contacting and non-contacting type - Self–generating and power operated. Compare their performance. |  |
| c) | Explain principle of operation of following: i) Spring ii) Diaphragm. |  |
| d) | Explain principle of operation of following transducers: i) Bellows ii) Flapper nozzle. |  |
| e) | Explain the following classification methods of transducers: - Electromechanical and opto-electrical - Self- generating and power operated. State one example of each. |  |
| f) | Explain the typical applications of measurement system in brief. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is meant by calibration? Explain terms: - Secondary calibration - Indirect calibration - Working standards. |  |
| b) | With help of block diagram, explain the buildings blocks of a measurement system. Clearly describe the basic functional elements and auxiliary elements. |  |
| c) | Explain the principle of working of: - Thermoelectric transducer - Electro-magnetic transducer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How can potentiometers be used in displacement measurement? With help of block diagram, showing output waveform at each stage, describe a practical method of displacement measurement. |  |
| b) | Explain principle of resistance strain gauge. What are the types of strain gauges? Compare the various types. Explain need for temperature compensation in such strain gauge applications. |  |
| c) | With block diagram, explain the different measurement techniques for strain measurement, showing output waveform at each stage. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of LDR in pressure and displacement measurement. |  |

………2/-

-2-

| b) | Explain the construction & characteristics of IC sensors. Describe a typical practical temperature measuring system, using 3 lead/4 lead method of compensation. |  |
| --- | --- | --- |
| c) | Describe the advantages, disadvantages and applications of capacitive transducers. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How do capacitive transducers attain variation in capacitance? Explain with neat figures. |  |
| b) | With help of block diagram, explain measurement of level using capacitive transducer. Sketch output waveform at each stage. |  |
| c) | What is a differential output arrangement? How is it useful? Explain with figures and examples. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the different ways in which inductance variation occurs in inductive transducers? Explain with figures and expressions. |  |
| b) | With block diagram explain measurement of pressure using inductive transducer. Draw output waveform at each stage. |  |
| c) | Explain the construction and operation of LVDT with figure. What is residual voltage? What are the applications of LVDT? |  |

***********