## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Automobile Engg.

Subject: Special Vehicles(5051)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are 4 x 2 and 4 x 4 trucks? |  |
| b) | What is a ‘foot valve’ of a pump? |  |
| c) | What is preventive maintenance? |  |
| d) | Why proper lubrication is essential for machine operation? |  |
| e) | List the special vehicles manufactured in India. |  |
| f) | What is a prime mover? |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the basic parts of a power shovel. Sketch different type of attachments to convert a power shovel into different types of machines. |  |
| b) | Explain with a neat sketch the power flow from engine to sprocket of a ripper dozer. |  |
| c) | How would you classify a bulldozer on the type of blade? Give a neat sketch of any one type with details. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | How are the graders classified? What are its main attachments and uses? |  |
| b) | Draw a neat sketch of a clamshell bucket and explain its construction. |  |
| c) | With the help of a neat sketch explain the construction of a scraper. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw a neat sketch and explain the construction of a Jack Hammer. |  |
| b) | How are pumps classified? Explain the construction of a centrifugal pump. |  |
| c) | Explain the different types of mixers and their uses. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Describe the construction of tractor brakes. |  |
| b) | With the help of a neat sketch, explain the clutch used in tractors. |  |
| c) | Draw a neat sketch of a turbocharger and explain its operation. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What maintenance action would you recommend for the following: i)Crawler Tracks ii) Transmission. |  |
| b) | Explain the procedure for carrying out inspection of an excavator. |  |
| c) | Describe the requirement for safe operation of cranes. |  |

***********