## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Elementary Ship Design(5331)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Explain any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Typical ship owner’s requirement |  |
| b) | Length by Breadth ratio of a ship |  |
| c) | Parent vessel in ship design |  |
| d) | Admiralty coefficient |  |
| e) | Light ship weight |  |
| f) | GRT and NRT |  |
| g) | Cubic number, square number |  |
| Q.No.2. | Answer the following: |  |
| a) | Explain in brief the process of ship design using design spiral. What are the parameters involved in design process? | (6) |
| b) | Explain the constraints on the dimensions length and breadth of a ship due to trade route. | (6) |
| Q.No.3. | A ship owner requires a vessel meeting the following requirements: - Dead weight =3250 tonnes - Service speed = 12.5 knots. Determine the length on waterline, breadth and draft of the vessel required to achieve the above, given Cd= 0.912 - 0.195 VL L=6.7 VV+22Δ13 Cb =1.137-0.33 VL B = L9 + 3.66 V is speed in knots and L in meters. | (12) |
| Q.No.4. | Answer the following: |  |
| a) | Explain how the lightship weight of a vessel can be estimated in the preliminary design stage. | (6) |
| b) | Estimate the grain capacity of a ship of 120m length 19m breath, depth 9.5m, draft 7.35m, CB=0.72,16(SA +SF)=0.65m, 12camber =0.18m, double bottom depth + coiling =1.07m Volumes Machinery space=2100m3 Cross bunker=150m3 Hatches =150m3 . | (6) …..2/- |

-2-

| Q.No.5. | Answer the following: |  |
| --- | --- | --- |
| a) | Explain the stability of a newly designed ship. | (3) |
| b) | Explain how the lines of a vessel are obtained given the sectional area curve of a vessel along with the associated load water line. | (9) |
| Q.No.6. | Draw a typical G.A. plan of a merchant vessel. Indicate the important machinery spaces and fittings on board the vessel. | (12) |
| Q.No.7. | Answer the following: |  |
| a) | What is freeboard? Explain broadly how the minimum freeboard is computed. | (6) |
| b) | Explain a typical technical specification of a merchant vessel prepared for the construction of a vessel. | (6) |

***********