## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg.

Subject: Illumination Engg.(5106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the points of distinction between luminance & illuminance. |  |
| b) | Define colour rendering effect. Arrange the following lamps in proper order starting with lamp having poor colour rendering effect to the one having best colour rendering effect: GLS Lamp, Mixed light lamp, HPSV lamp, HPMV lamp. |  |
| c) | State the factors effecting Coefficient of Utilization. |  |
| d) | Define the following terms with reference to street lighting. 1) Kerb 2) Outreach 3) Overhang. |  |
| e) | What is the basis for classification of flood light projectors? Hence classify them. |  |
| f) | Give any three special features of auditorium lighting. |  |
| g) | Give any three special features of hospital lighting. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enumerate all the factors affecting good lighting. “Hard shadows are to be minimized or avoided but soft shadows are to be preserved” - say true or false & justify your choice. |  |
| b) | State the different lighting systems and highlight each system with the help of neat sketches. |  |
| c) | A twin tube luminaire with diffuser is installed in a room. The dimension of diffuser is 150mm x 1250 mm. The Luminaire is viewed in transverse direction from a distance of 4 m, considering the mounting height of luminaire as 3 m, with an eye level of 1.5 m. Determine the brightness of working plane, given the absorbtion factor of luminaire as 60% with 40 watt fluorescent tube contained within a luminaire. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define 1) M.H.C.P. 2) M.S.C.P 3) M.H.S.C.P & 4) C.P. State the laws of illumination and give one application of each law. |  |
| b) | What is stroboscopic effect? State the methods of minimizing it & explain any one of such method of your choice in brief with the help of neat circuit diagram. Is it different from flickering? |  |
| c) | What are the advantages of polar curve? State all the steps in plotting polar curve. |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | List out all the steps involved in indoor lighting design. State the basic requirements for such lighting design. Also state the factors & ratios involved in such design & give nominal values of each ratio & factor. |  |

………2/-

-2-

| b) | A plate inspection steel plant 100m long, 50m wide & 20 m height is to be illuminated with an average illumination level of 500 Lux. The reflection factor of ceiling is 50%, wall is 50% and floor is 10%. If 400 watts sodium vapour lamp is to be provided, determine the actual number of luminaires required to be installed. Also give the layout of the arrangement. C.O.U table is enclosed. |  |
| --- | --- | --- |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | With reference to flood lighting: - What do you understand by aiming of flood light projector? Explain it in brief with neat diagrams. - What factors are to be considered in the design? Also, state the basis on which such factors are selected. Give their range of nominal values. - Give the distance of mounting of each type of flood light projector. | (4) 2+2+2 (2) |
| b) | Design an economical flood lighting scheme for a storage yard of 30m x 15 m, which is to be provided an illumination level of 150 Lux using steel tubular poles of 12m mounting height. The flood light fittings of following photometric data are available. Round fittings with 250 watts HPSV lamp having beam angle of 2 x 25, Beam factor = 0.5. Square fitting with 2 x 250 W HPSV lamp having beam angle of 2 x14 & 2 x 40, Beam factor of 0.6. |  |
| Q.No.6. | Answer any one of the following Questions: | (12) |
| a) | With reference to street lighting: - Give the fundamental arrangements for siting of luminaries. Also give advantages of each. - How do you select the mounting height for street lighting Luminaires? - State factors affecting choice of street light luminaries. - What is the basis for providing spacing between street light Luminaries? Also give their nominal values. | (6) (2) (1) (3) |
| b) | A Road with a Carriage way width of 9 m is to be illuminated using 250W, HPSV lamp in Semi cut off Luminaire. Select: 1)Arrangement of Luminaire 2) Mounting height 3)Spacing between Luminaire 4) Outreach & Overhang Also, calculate average Illumination level. The polar curve data is as given below: S/HM= 1.25 Rom reflectance; Rom Index. Ceiling; Wall; Floor; 0.75; 1.00; 1.25; 1.5; 2.0. C.O.U. 0.75; 0.5; 0.1; 0.51; 0.57; 0.62; 0.65; 0.69. 0.75; 0.3; 0.1; 0.46; 0.53; 0.58; 0.62; 0.66. 0.75; 0.1; 0.1; 0.43; 0.5; 0.55; 0.59; 0.64. 0.5; 0.5; 0.1; 0.50; 0.56; 0.61; 0.64; 0.68. 0.5; 0.3; 0.1; 0.46; 0.53; 0.58; 0.61; 0.65. 0.5; 0.1; 0.1; 0.43; 0.50; 0.55; 0.58; 0.63. | (2) (2) (2) (2) (4) |
|  | angle; 0; 10; 20; 30; 40; 50; 60; 70; 80; 90; 100; 105. C.P in cd; 1000; 1500; 1650; 2250; 2800; 3200; 3400; 3200; 2750; 1300; 800; 500. |

***********