## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics Engg./EC

Subject: Communication Systems-I(4150)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe thermal noise. |  |
| b) | Describe three differences between AM and FM. |  |
| c) | Draw an over-modulated AM signal and explain the drawbacks of over-modulation. |  |
| d) | Describe in brief the process of grid modulation. |  |
| e) | Define:- i) Signal-to-noise ratio ii) Noise figure. |  |
| f) | Describe AFC. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | An AM transmitter radiates 10 KW of power when the carrier is unmodulated and 12 KW when the carrier is sinusoidally modulated. Calculate the modulation index. Also calculate the total modulation index and total radiated power if another sine wave with 40% modulation is transmitted simultaneously. |  |
| b) | Describe a super-heterodyne radio receiver with the help of a block diagram. |  |
| c) | Draw the block diagram of a stereophonic FM receiver and explain the same. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Draw the circuit of an FET-based balanced modulator and explain mathematically, how it suppresses the carrier. |  |
| b) | Describe the phase-shift method of sideband suppression with the help of a block diagram. |  |
| c) | Describe in brief: - Vestigial sideband transmission - The frequency bands used for radio communication. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Draw a block diagram that depicts how frequency modulation is done using the Armstrong method. |  |
| b) | Describe the basic reactance modulator for frequency modulation with the help of a diagram. |  |
| c) | What is pre-emphasis? Why is it used? How is the effect of pre-emphasis overcome in receiver? Draw circuit and pre-emphasis curve to explain the same. |  |
| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| a) | With the help of a block diagram, describe the transmission of a stereophonic FM signal. |  |
| b) | Draw the circuit diagram of a balanced slope detector and explain how an FM signal is demodulated. |  |
| c) | Describe a phase discriminator and list its advantages. |  |

………2/-

-2-

| Q.No.6. | Answer any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Draw the block diagram of a basic FM receiver and explain. |  |
| b) | Describe what is AGC, why it is used and what are the different types of AGC. |  |
| c) | What is super heterodyne tracking? Explain in brief. |  |
| d) | Differentiate between low-level and high-level amplitude modulation. |  |

***********