## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-II(4062)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define traction and tractive effort of an automobile. |  |
| b) | Define and state or derive the relation for gradiability of a vehicle. |  |
| c) | Describe rolling, pitching and bouncing motions in an automobile. |  |
| d) | State any three important requirements of a good car body. |  |
| e) | Explain the purpose of camber angle in front wheel geometry. |  |
| f) | State the advantages of fluid flywheel over a friction clutch. |  |
| g) | List the advantages Mc Pherson strut suspension over the double - wishbone type. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The engine of a jeep is known to be able to provide 41.25 KW for propulsion purposes. In a certain application, the jeep weighing 12.70 KN is required to pull a trailer of gross weight 10.88 KN at a speed of 57.75 km/hr in top gear on level. The resistance to motion is given by the equation R=aW+bV2, where a=0.016 and b=0.056 x 10-3, W is in KN and V in Km/hr. Find out if the jeep is adequate for the job, if the transmission efficiency is 90%. What is the pull in the coupling at this speed? If the available power is just utilized in top gear by suitably loading the trailer, what is the pull in the coupling at 57.75 Km/hr? |  |
| b) | The coefficient of rolling resistance for a truck weighing 63.50 KN is 0.018 and the coefficient of air resistance is 0.00281 x 10-2 in the formula R=KW+KaAV2 KN, where A is m2 of frontal area and V the speed in Km/hr. The transmission efficiency in top gear of 6.2: 1 is 90% and that in the second gear of 15:1 is 80%. The frontal area is 5.574m2. If the truck has to have a maximum speed of 88 Km/hr in top gear, calculate. - The engine B.P. Required; - The engine speed if the driving wheels have an effective diameter of 81.25cm; - The maximum grade the truck can negotiate at the above engine speed in second gear; and - The maximum drawbar pull available on level at the above engine speed in second gear. |  |

………2/-

-2-

| c) | A truck weighs 102KN and the engine develops 97 KW at 2400 r.p.m. The transmission efficiency is 90% in top gear of 3.4:1 and 85% in third gear of 8.4:1 The performance of the vehicle is such that it will just reach a speed of 86.8 Km/hr at 2400 r.p.m. at wide open throttle when running on the level in still air, and at the same engine speed in third gear it will jut climb a gradient of 1 in 14. If the total resistance in KN is given by the formula. R= KW+KaAV2+W sin θ Where A is in m2; V in Km/hr and W in KN, calculate K and Ka and hence the engine B.P. required for climbing a grade of 1 in 40 at 48 Km/hr in top gear. How much more weight can be added to the vehicle to use the engine power fully under the above condition. |  |
| --- | --- | --- |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of worm and nut type steering gear box. |  |
| b) | With the help of a neat sketch, explain the working of integral type hydraulic power steering. |  |
| c) | A track has pivot pins 137 cm apart, the length of each track arm is 18 cm and the track rod is behind the front axle and127 cm long. Determine the wheelbase, which will give true rolling for all wheels when the car is turning so that the inner wheel stub axle is 60 to the centre line of the car. A geometrical construction may be used. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw a neat sketch showing the graph of % slip versus speed of driving member of a fluid flywheel & explain. State the disadvantages of a fluid flywheel. |  |
| b) | With the help of a neat sketch, explain the working of an overdrive of an automobile. |  |
| c) | Explain the working of shift valve in an automatic transmission. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Distinguish between leaf spring and coil spring. |  |
| b) | What are the advantages of independent suspension over rigid axle suspension? |  |
| c) | With the help of a neat sketch, explain the working of vertical guide suspension. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | State the advantages of sub-frame body structure over the fully unitary construction. |  |
| b) | Discuss the design of automobile body with respect to aerodynamics and visibility. |  |
| c) | With the help of a neat sketch explain the adjustment/ operation of luxury seat mechanism. |  |

***********