## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Chemistry of Welding Processes(4231)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following: | 10x1.5=15 |
| --- | --- | --- |
| a) | State any three advantages of using liquid oxygen. |  |
| b) | Why is Helium better suited as a shielding gas for welding underside of a job? |  |
| c) | How is Acetylene checked for the presence of Hydrogen Sulphide impurity? |  |
| d) | Define calorific value. |  |
| e) | State the types of combustion on the basis of flame velocity. |  |
| f) | How is flux selected for soldering? |  |
| g) | Mention the flux used for welding Aluminium & its alloys. |  |
| h) | State the disadvantages of using bare wire electrodes. |  |
| i) | What are the typical faults caused by gas absorption? |  |
| j) | Write the formula for calculation of carbon equivalent for alloy steel. |  |
| Q.No.2. | Answer any three of the following: | 3 x 4 = 12 |
| a) | Explain in brief Lindes process of production of Oxygen in liquefaction of air. |  |
| b) | What are oxides? With chemical equations explain the basis for the classification of acidic and basic oxides. |  |
| c) | Explain the significance of oxidation and reduction in welding. |  |
| d) | Explain the need for preheating carbon dioxide during welding. |  |
| Q.No.3. | Answer any three of the following: | 3 x 4 = 12 |
| a) | Draw a neat labelled sketch of Carbide to Water Acetylene generator and explain the process in brief. |  |
| b) | Write a note on explosibility of pure Acetylene. |  |
| c) | What is the need for the use of porous fillers in Acetylene cylinders? State the requirements of porous fillers. |  |
| d) | State the properties and applications of any two fuel gases. |  |
| Q.No.4. | Answer any three of the following: | 3 x 4 = 12 |
| a) | Explain the structure and chemistry of Oxy-Acetylene flame. |  |
| b) | How is efficiency of welding affected by (i) Flame Angle (ii) Metal thickness. |  |
| c) | What is the basic difference between: - Welding and Brazing - Brazing and Soldering. |  |
| d) | State the necessity and functions of a brazing flux. |  |

………2/-

-2-

| Q.No.5. | Answer any three of the following: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Discuss the various flux ingredients and their functions. |  |
| b) | Write a note on care and storage of electrodes. |  |
| c) | What are the effects of various gases absorbed in the weld pool? |  |
| d) | Explain any two methods of preventing gaseous contamination of molten weld pool. |  |
| Q.No.6. | Answer any three of the following: | 3 x 4 = 12 |
| a) | Distinguish between Metals and Non-metals. (any four points) |  |
| b) | Draw the following crystal lattice structures: (i) F. C. C. (ii) B. C. C. |  |
| c) | State any two industrial applications each of Copper, Aluminium, tin and lead. |  |
| d) | What is meant by “Chilling of Flame”? |  |

***********