## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Microprocessor Based Instrumentation(5114)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How is a 20 bit physical address generated in 8086? |  |
| b) | Explain the difference between MOV AX, 5000H and MOV AX, [5000H] |  |
| c) | Name the registers used for math operations, give example and write format for PSW. |  |
| d) | Explain the instruction queue used in 8086. |  |
| e) | Compare the following processor with reference to data lines, address lines, speed & compatibility of instruction sets: 80186, 80286 and Pentium processors. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the role of two functionally independent sections of 8086 CPU. |  |
| b) | What is memory segmentation? What is the role of Segment Register in 8086? |  |
| c) | Compare between the features of microcontrollers 8031 and 8051. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the timer mode operation of 8051 microcontroller. |  |
| b) | Draw the internal block diagram of 8051 and list out all the features of this microcontroller. |  |
| c) | Draw and explain I/O port pin configurations for port 0 & port 1. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Explain and write the format for IE (Interrupt enable) and IP (Interrupt priority) special function registers. |  |
| b) | Explain the serial data transmission modes for mode 0 & mode 1. |  |
| c) | Explain the 8051 oscillator and clock circuitry. |  |
| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| a) | Write a program to reverse a block of data bytes stored at internal RAM memory location 00H and the size of the block of data byte is ten which is stored in location 1FH. |  |
| b) | Write a program to search for data byte FEH & increment it by one, the data bytes are stored in RAM location starting from 10H to 18H. |  |
| c) | Explain the relative range, short absolute range and long range with reference to 8051. Also list any five jump instructions. |  |

………2/-

-2-

| Q.No.6. | Answer any two: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the following instructions with an example: - MOV DPTR, #nn - MOV @Rp, #n - XCHD A, @Rp - ADDC A, add - RLC A - CJNE A, #n, radd |  |
| b) | Write the contents of Register A after execution of each instruction & mention the status of the carry and overflow flag for the following program: MOV A, #1CH MOV RS, #OAH ADD A, RS ADD A, RS ADDC A, #10H ADDC A, #10 H |  |
| c) | Briefly explain the program counter and data pointers, which are the 16-bit registers of 8051. |  |

***********