## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Materials & Structures(4232)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Esteel = 2.0 x 105 N/mm2

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What will be the deformation if a rod 1m long, 20mm x 20mm in cross–section is subjected to deformation due to self weight. Density of steel= 7860 Kg/m3. |  |
| b) | What is complimentary shear? Explain with a neat sketch. |  |
| c) | Define rigid, plastic & elastic body. |  |
| d) | Define i) Resilience ii) Proof Resilience iii) Modulus of Resilience. |  |
| e) | Write equations to show relation between E, K & C. |  |
| f) | Calculate the strain energy stored in a bar 2000mm long, 50mm wide, 40mm thick subjected to a tensile load of 60 KN. |  |
| g) | Determine MI of a triangular section having 80 mm width (base) and 60 mm height about its C.G and its base. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The rod of a hydraulic lift is 12m long & 32mm in diameter. It is attached to a plunger 100mm in diameter working under a pressure of 8N/mm². Find alteration in the length of the rod. |  |
| b) | A copper bar 36mm diameter is rigidly attached at both ends to the inside of a steel tube 50mm external diameter & having 5mm thickness. It is subjected to a pull of 100 KN. Find the stress induced in each metal on 1.5m length. Ec= 1.1x10N/mm² |  |
| c) | A bar of steel is 500mm long. The two ends are 32mm & 28 mm in diameter respectively & 125mm long each. The middle portion is 25mm in diameter. Determine the contraction in the length if the bar carries an axial thrust which produces a stress of 100N/mm² in the smallest section. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel rod 36mm in diameter & 4.50m long is heated through 40C above atmospheric temperature when the ends are clamped. On cooling back to atmospheric temperature, it is found that the clamps have yielded by 0.5mm. Find the pulls exerted on the bar by the end clamps. =10x10-6/C. |  |
| b) | A 100mm x 100mm x 12mm bar of Tee section is 4m long. An axial load of 180KN is suddenly applied to it. Find (1) The maximum instantaneous stress (2) The maximum instantaneous elongation. |  |
| c) | Determine the greatest weight that can be dropped from a height of 200mm on to a collar at the lower end of a vertical bar, 32mm in diameter & 2.4m long without exceeding the elastic limit of 280N/mm². Collar is made of steel. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw SFD & BMD for an overhanging beam ABC, supported at A & B, l(AB)=6m, l(BC) overhang = 2m, having a clockwise couple of 200KN-m, at a point 2m from A and a 100KN-m clockwise couple at the point C. |  |
| b) | A S.S. beam of 5m span carries a gradually varying load of Zero at one end & 5 KN/m at the other. Draw SFD & BMD, considering a self weight of 0.5 KN/m. |  |
| c) | A cantilever 1.5m long is loaded with a u.d.l of 2KN/m over a length of 1.25m from the free end. It has a point load of 2KN at the centre. Draw SFD & BMD. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine MI of an I section, 250mm x 12mm flanges & web 250mm x 100 mm. |  |
| b) | A cast iron test beam 25mm square in section is broken by a load of 750 N applied at the centre of a span of 1m. Using a factor of safety of 8, Calculate the safe u.d.l for a beam 120mm wide & 300mm deep, freely supported over a span of 5m. |  |
| c) | Calculate the width & depth of the strongest beam of rectangular section that can be cut out of a cylindrical log of wood whose diameter is 300mm. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A beam of symmetrical section is 350mm deep & has a MI of 13160 x 10 mms about its principal axis. To what radius may it be bent if the maximum skin stress is not to exceed 126N/mm². What is the Moment of Resistance? Beam is made of steel. |  |
| b) | A beam of triangular section having a base width of 100mm & height 150mm is subjected to 15KN shear force. Find the value of maximum shear stress & Sketch the shear stress distribution along the depth of the beam. |  |
| c) | An unequal angle 150mm x 75mm, thickness of metal 8mm with longer vertical log used as a beam carries a load of 6KN/m over a span of 3m. Find the maximum intensity of shear stress & sketch the distribution of stress intensity across the section. |  |

***********