## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: D.C.Machines(4109)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the parts of a DC machine. State the functions of any two parts. |  |
| b) | Define : i) Pole pitch ii)Front pitch iii)Commutator pitch. |  |
| c) | State the possible reasons for failure of self excited d.c. series generator to build up voltage. |  |
| d) | Draw a neat labelled diagram of third brush generator and state 2 of its applications. |  |
| e) | Briefly explain the need for parallel operation of d.c. generators. |  |
| f) | Briefly explain the working principle of a d.c. motor. |  |
| g) | Define: i) Armature reaction ii) Commutation iii) Interpole. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With necessary diagrams explain the working principle of a simple loop generator and the production of unidirectional current in its external load circuit. |  |
| b) | A 8 pole d.c. shunt generator with 778 wave connected armature conductors running at 500 rpm supplies a load of 12.5 resistance at a terminal voltage of 250V. The armature resistance is 0.24 and field resistance is 250. Find out the induced emf, flux per pole and power developed. |  |
| c) | Draw and explain the various characteristics of a self excited d.c. shunt generator. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | The magnetisation curve data of a d.c. shunt generator at 1500 rpm is as follows: If (A) 0 0.4 0.8 1.2 1.6 2 2.4 2.8 3.0 Eo(V) 6 6 120 172.5 202.5 221 231 237 240 For this generator i) Find the no load emf induced for shunt field resistance of 100 ii) Draw the magnetisation curve for speed of 1700 rpm and therefrom the no load induced emf for a field resistance of 100 . |  |
| b) | A d.c. shunt generator delivers 195 A at a terminal voltage of 250V. The armature and shunt field resistances are 0.02 and 50 respectively. The iron & frictional losses equal 950 W. Find: i) generated emf ii) copper losses iii) Prime mover output iv) commercial efficiency. |  |
| c) | With neat diagram explain the construction & working principle of a bipolar d.c. welding generator. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With neat diagram explain the process of connecting one d.c. series generator in parallel with another already operating d.c. series generator and the need of equaliser bars for satisfactory parallel operation of d.c. series generators. | ………2/- |

-2-

| b) | With neat diagrams explain the classification of d.c. motors based on the connection of field and armature windings and write the voltage and power equation for each of them. |  |
| --- | --- | --- |
| c) | A 4 pole, 240V wave connected shunt motor gives out 11.19 KW when running at 1000 rpm and drawing armature and field currents of 50A and 1A respectively. The armature resistance is 0.1 and its has 540 conductors. Assuming drop of 1V/brush find: i) Armature torque ii) Shaft torque iii) Iron & rotational loss and iv) efficiency. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With neat diagram explain the Ward-Leonard method of speed control of d.c. shunt motor. State its merits and demerits. |  |
| b) | Draw and explain the various performance characteristics of a d.c. series motor. |  |
| c) | Write short notes on the following: |  |
| i) | Significance of back emf in a d.c. motor |  |
| ii) | Necessity of starter for starting a motor |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With neat diagram explain the construction & working of a 4 point starter for d.c. shunt motor and functions of each of its parts. |  |
| b) | With neat diagram explain the concept of armature reaction and its effects on the operation of a d.c. machine. List the methods that can be adopted to reduce each of these effects. |  |
| c) | With neat diagram explain the principle of commutation and reasons for poor commutation. Briefly explain any one method for improving commutation. |  |

***********