## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Drawing(4234)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Structural Engineers Handbook SP:6(1) & IS 800 is permissible.

5) All dimensions in the figures are in ‘mm’ unless specified.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Determine the following of the steel rolled section ISBA 275@ 40.9 kg/mt: i) tf ii) tw iii)Zyy. |  |
| b) | Give the symbols of the following: |  |
|  | i) Site rivet counter-sunk for side |  |
|  | ii)Shop HSFG bolt |  |
|  | iii)Open hole counter-sunk near side. |  |
| c) | Explain the importance of reference line in symbolic weld representation. |  |
| d) | Sketch a typical Ridge joint. |  |
| e) | Sketch a copper corrugated fitting and explain its working. |  |
| f) | Sketch a clamp type Piper hanger. |  |
| g) | Sketch the cross-section of any one of the following rolled steel section, indicate dimensions clearly. |  |
|  | i) ISWB 600 @ 133.7kg/mt ii) ISA200150 x 15.0mm |  |
| Q.No.2. | Draw the Fabrication Drawing of the Gusset plates at ‘A’ & ‘B’ and determine the exact length of the member A-B, in the portion of the truss, whose design drawing is shown in Fig.1 | (12) |
|  | ### NOTE FOR FIG.1 |  |
|  | 1. All members used are ISA 6060 x 5.0 mm. |  |
|  | 2. Gusset plate thickness used is 6.0 mm. |  |
|  | 3. Min. weld length of 5.0 mm fillet leg used is 200 mm to |  |
|  | connect each member to the gusset plate. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | Draw the Isometric view and Developed view of the portion of the pipeline, whose Top view is shown in Fig.2. (Assume suitable pipe lengths wherever not mentioned). | (8) |

………2/-

-2-

| b) | Differentiate between Fabrication clearance and Erection clearance. Also differentiate between Design drawing and Fabrication drawing. | (4) |
| --- | --- | --- |
|  | #### OR |  |
| b) | Sketch a corner column to beams riveted connection. | (4) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Draw the Top view, Front view and L.H.S.V of the support stucture whose pictorial line diagram is shown in Fig.3. | (8) |
|  | ### NOTE FOR FIG.3 |  |
|  | 1. All members used are ISA 4545 x 4.0 mm. |  |
|  | 2. All joints are welded joints, Butt or fillet. No gusset plates at the joints are used. |  |
| b) | Make a Bill of Material (BOM) of the support structure, shown in Fig.3, Q.4a. | (4) |
|  | OR |  |
| b) | Explain the main essential columns of a Bill of material for a structure. | (4) |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch a Double Riveted Zig-Zag Lap Joint between two plates of thickness 10mm and 12mm. Indicate dimensions clearly. |  |
| b) | Sketch a Column Splicing of Bottom Column ISMB200 @ 25.4 Kg/mt and Top Column ISLB200@19.8 Kg/mt. Indicate dimensions clearly. |  |
| c) | Give the Symbolic weld representation of the following: (any two) |  |
|  | i) Double Bevel Butt, welded all around a part to be done at site. |  |
|  | ii) Single V-Butt with wide root face of size 6 mm all along |  |
|  | the joint to be welded on other side. |  |
|  | iii) Square Butt weld ground flush of size 8 mm to be welded |  |
|  | on both sides of the plate. |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Procedure involved in converting a Design drawing of a riveted structure, into its Fabrication drawing |  |
| b) | Importance of Developed view of a pipeline |  |
| c) | Specifications of different steel rolled sections |  |

……3/-