## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg./A

Subject: Theory of Machines(4039)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define number of degrees of freedom. What is the number of degrees of freedom obtained when one link of a five bar chain is fixed? |  |
| b) | What is fluctuation of energy as related to flywheel? |  |
| c) | Define effort and power of a governor. |  |
| d) | What is creep in a belt drive? |  |
| e) | What is a dynamometer? Mention any four types of dynamometers. |  |
| f) | Why is roller follower preferred over a knife-edge follower? Mention any two motions, which can be imparted to a follower by a cam. |  |
| g) | Mention any three applications of gyroscope. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | In the mechanism shown in fig.1, the crank OA is 100mm long and rotates at 100 r.p.m. clockwise. The straight link BCD rocks about the fixed point C. The length BC and CD are 200 mm each and the link AB is 300 mm long. The slider E is driven by the rod DE which is 250mm long. Determine the velocity and acceleration of E. | (08) |
| b) | For the mechanism shown in Fig.2, determine the velocity and acceleration of the slider B by approximate analytical method. Links OA and AB are 160mm and 850mm long respectively and link OA rotates at a uniform speed of 150 r.p.m. counter– clockwise. | (04) |
|  | ### OR |  |
| b) | A flywheel is running at 180 r.p.m. and after 20 seconds, it runs at 140 r.p.m. How many revolution will it make, and what time will elapse before it stops, if the retardation is uniform? | (04) |
|  |  | ….2/- |
|  | -2- |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The spindle of a screw jack has single start square thread with outside diameter of 45mm and a pitch of 10mm. The spindle moves in a fixed nut. The load is carried on a swivel head but not free to rotate. The bearing surface of the swivel head has a mean diameter of 60mm. The coefficient of friction between the nut and screw is 0.12 and that between the swivel head and the spindle is 0.1. Calculate the load that can be raised by an effort of 200 N applied at the end of a lever of effective length 350mm. Also determine the efficiency of the lighting arrangement. |  |
| b) | In a thrust bearing, the external and internal radii of the contact surfaces are 210mm and 160mm respectively. The total axial load is 60KN and coefficient of friction is 0.05. The shaft rotates at 380 r.p.m. The intensity of pressure is not to exceed 350 KN/mm2. Calculate - Power lost in friction, and - Number of collars required. |  |
| c) | With the help of a neat sketch, explain the construction and working of Hartung governor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A leather belt is required to transmit 7.5 KW from a pulley 1.2m in diameter running at 250 r.p.m. The angle of lap is 165º and the coefficient of friction between the belt and the pulley is 0.3. If the safe working stress for the leather belt is 1.5 MPa, density of leather is 1 Mg/m3 and the thickness of the belt is 10mm, determine the width of the belt considering centrifugal tension. |  |
| b) | A flat belt, 240 mm wide and 14mm thick, transmits power from one pulley to another. The ratio of driving tension is 2.1, maximum permissible stress in the belt material is 2.2 MPa and mass density of the belt material is 1Mg/m3. Determine the speed at which the power transmitted is maximum and the maximum power transmitted. |  |
| c) | With the help of a neat sketch, explain the construction and working of Whitworth quick return motion mechanism. |  |
| Q.No.5. | Answer the following: |  |
| a) | A shaft has four masses M1, M2, M3 and M4 attached to it. They all revolve in the same plane. The magnitudes of the masses are 6kg, 5kg, 9kg and 7kg respectively. They are located at radial distances of 100 mm, 125mm, 150mm and 75mm from the axis of the shaft respectively. The angular positions of M2, M3, and M4 from M1 are 60º, 135º and 270º respectively. Determine the position and magnitude of mass M5 to revolve at radius of 350 mm in order to balance the system. | (03) |

………3/-

-2-

|  | ### OR |  |
| --- | --- | --- |
| a) | What is cycle, frequency and period of vibration? | (03) |
| b) | A Cam operating a knife-edge follower has the following data: - Follower moves outward through 40mm during 60º of cam rotation with uniform velocity. - Follower dwells for the next 45º of cam rotation. - Follower returns to its original position during next 90º of cam rotation with equal and uniform acceleration and retardation - Follower dwells for the rest of cam rotation. The least radius of the cam is 50mm. The axis of the follower is offset to the right side of the camshaft axis by 20mm. Draw the profile of the cam if it rotates clockwise at a speed of 200 r.p.m. Calculate the maximum velocity and acceleration of the follower during its return stroke. | (09) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A differential band brake is shown in fig.3. The diameter of the drum is 800mm. The coefficient of friction between the band and drum is 0.3 and the angle of lap is 240º. When a force of 600N is applied at the free end of the lever, find for clockwise rotation of the drum - The maximum and minimum tension in the band - The torque which can be applied by the brake. |  |
| b) | With the help of a neat sketch, explain the construction and working of belt transmission dynamometer. |  |
| c) | - The gearing of a machine tool is shown in fig.4. The motor shaft is connected to gear A. The gears B, C, D and E are fixed to parallel shafts rotating together. The gear F is fixed to the output shaft. The number of teeth on each gear are as below: Gear A B C D E F No. of teeth 20 50 25 75 26 65 What is the train value? Determine the speed of gear F if the motor shaft rotates at 975 r.p.m. ii) Draw the profile of two teeth of a rack to mesh with involute gear of module 6mm and pressure angle 14½. | (02) (04) |

…….4/-