## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Architectural Engg.

Subject: Estimation & Costing(4529)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five in short: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is meant by Estimation? |  |
| b) | What is carpet area? |  |
| c) | What is meant by item of works? |  |
| d) | What is measurement sheet used for? |  |
| e) | Mention market rates of any 3 different construction material. |  |
| f) | Market rates of any item of works fluctuate. Why? |  |
| Q.No.2. | Answer any three of the following: | 3 x 6 = 18 |
| a) | Distinguish between cubic content estimate and carpet area estimate. |  |
| b) | Prepare a statement of Plinth Area estimate for a residential house (G+1). Assume all necessary data. |  |
| c) | Explain the different situations when estimation and costing is required in a construction project. |  |
| d) | Explain the features of an item-wise estimate. |  |
| Q.No.3. | Answer any three of the following: | 3 x 6 = 18 |
| a) | Gives the title and unit of measurement of any six items of works. |  |
| b) | Differentiate between Schedule of Rates and Bill of Quantities. |  |
| c) | Write the specifications for: i) Internal Plaster ii) Flooring. |  |
| d) | Prepare a statement on sanitary estimate for the toilet shown in figure ‘A’. Assume necessary data. |  |
| Q.No.4. | Prepare the Measurement sheet and Abstract sheet for any 2 of the following as indicated in figure ‘A’. | 2 x 6 = 12 |
|  | i)Beams and slab – R.C.C |  |
|  | ii)R.C.C. items of staircase |  |
|  | iii)Painting – external wall and window grills. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Which factors can affect the rate analysis of R.C.C. items of works of a Building? |  |
| b) | Prepare the rate analysis for a 23cms thick Laterite masonry wall in cement mortar 1:3. Assume necessary data and rates if required. |  |
| c) | Prepare rate analysis for supply and fixing of Indian W.C. in the Toilet. Assume any additional data and rates as required. |  |

……..2/-