## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Automobile Engineering

Subject: Hydraulic Machinery(4053)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the term : i) Specific weight. ii) Specific gravity. |  |
| b) | Explain with mathematical expression the relationship between gauge pressure and absolute pressure. |  |
| c) | State two important characteristics of Laminar flow. |  |
| d) | What do you understand by the term: ‘Total pressure acting on an immersed surface’. |  |
| e) | State the functions of a Foot valve and Strainer. |  |
| f) | Differentiate between a gear pump and a vane pump. |  |
| g) | Explain compressibility of a fluid. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the working of Bourdon pressure gauge. |  |
| b) | Discus the conditions under which ‘cavitation’ and ‘negative slip’ occur and state why air vessels are used in reciprocating pumps. |  |
| c) | Describe with the help of a neat sketch the working of hydraulic intensifier. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the various methods adopted to increase the efficiency of a centrifugal pump by altering the shape of the casing of chamber surrounding the impeller. |  |
| b) | A triangular gate which has a base of 1.5m and an altitude of 2m lies in a vertical plane. The vertex of the gate is 1m below the surface of a tank which contains oil of specific gravity 0.8. Find the force exerted by the oil on the gate and the position of the centre of pressure. |  |
| c) | Describe with the help of neat sketch the construction and working of a plunger pump. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the discharge through 60º triangular notch, when the head is 0.16 m. Take Cd = 0.6. |  |
| b) | The difference in head between two points of a pipe 300m long is 34cm. The diameter of the pipe is 25cm. Taking the Darcy coefficient as 0.008 and neglecting minor losses, calculate the discharge through the pipe. |  |
| c) | A 0.3m pipe carries water at a velocity of 24.4 m/s. At points ‘A’ and ‘B’ measurements of pressure were 361 KN/m2 and 288 KN/m2 respectively and elevation at ‘A’ is 30.5m and at ‘B’ is 33.5m. Find the loss of head between ‘A’ and ‘B’. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a neat sketch explain the construction and working of hydraulic jack. |  |
| b) | Differentiate between a centrifugal pump and a reciprocating pump considering atleast four aspects. |  |
| c) | A jet of water from an orifice 25mm diameter under a constant head of 1.5 m falls 0.915m vertically before it strikes the ground at a distance of 2.288m measured horizontally from vena-contracta. Find Cv. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Laws of Fluid friction |  |
| b) | Multi-stage pumps |  |
| c) | Micro manometer |  |
| d) | Orifice meter |  |

***********