## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Control Engineering(4219)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between Regulated and Followup Feedback systems. |  |
| b) | Explain the basic structure of a DC machine. |  |
| c) | If x1(t)=1 and X2(t) = e –2t , find L [4x1(t) + 3x2(t)]. |  |
| d) | What are the features of servomotor? List few applications. |  |
| e) | List out the disadvantages of the static error coefficient method. |  |
| f) | Draw the Bode magnitude plot for G(s) = S. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the block diagram and explain the control system involved in controlling the speed of a car by a driver. |  |
| b) | Define control system. Draw and explain the block diagram of a multivariable control system. |  |
| c) | Describe the working of a Synchro control receiver/motor with the help of a schematic diagram. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Using characteristic curve of ideal Potentiometer, explain the linearity of the device. |  |
| b) | Differentiate between A.C and D.C. tachometer. Explain one application of D.C. tachometer in brief. |  |
| c) | Find the transfer function of potentiometer and DC tachometer. |  |
| d) | Assume any four different Pole locations for a system. Sketch the response and comment on stability in each case. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | If the system transfer function is y(s)x(s)=(s+4)(s2+2s+5) obtain the differentiate equation representing the system. |  |
| b) | Obtain the transfer function for the system represented in fig.4.2 and draw the reduced block diagram. |  |
| c) | Obtain the steady state error of a type – 0 system to standard test signal inputs. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A second order system has the constant kp=0.1, ka=100, h=1, Km/J = 4 and F/J = 2. Determine the values of , wn and comment on the stability showing response. |  |
| b) | Draw the waveform and explain each specification of the 2nd order system time response (underdamped) to step i/p. |  |
| c) | Sketch the typical Polar plots for the following: i)G(s)=k(1+sT1)(1+sT2) ii)G(s)=1s(1+sT1) iii)G(s)=1s2(1+sT1)(1+sT2) |  |
| Q.No.6. | Answer the following: | 2 x 6 = 12 |
| a) | For the closed loop system shown in fig. 6.1, determine the range of R for the system to be stable. |  |
| b) | Sketch the bode plot for a unity feedback control system which has transfer function G(s)=s+10s(s+100). Determine the Gain cross over frequency. | …..3/- |