## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

#### Programme: Mechanical Engg./P

Subject: Machine Design Practice(4045)

# Time Duration: 4 Hrs.                                                Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is stress concentration? Define theoretical stress concentration factor. With neat sketches show different method of reducing it. |  |
| b) | What do you mean by fatigue failure? Draw S-N diagram showing endurance limit and specimen with dimensions used for endurance test. |  |
| c) | Why are elastic theories of failures used in Machine Design? List the different theories of failures. Explain in brief Tresca’s theory. |  |
| d) | Explain in brief conditions i) self locking of screws ii) overhauling of screws. |  |
| e) | Compare antifriction bearings with journal bearings. |  |
| f) | What is the function of cotter in a cotter joint? Why taper is provided to the cotter? What is its normal value? |  |
| g) | What are functions of spring? List different types of spring. Recommend any two materials for spring. |  |
| h) | State the meaning of the following grades of steel i) Fe 500 ii)C30 iii) 30Cr13 iv) Fe E250. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a knuckle joint to connect two mild steel rods to sustain an axial pull of 150KN. The pin and the rods are made of same material. Assume the working stresses in the material as 80MPa in tension, 40 MPa in shear, 120 MPa in compression. |  |
| b) | The pull in the tie rod of an iron roof truss in 50KN. Design a suitable Turnbuckle. The permissible stresses are 75MPa in tension, 38MPa in shear and 90MPa in crushing |  |
|  |  | ….2/- |
|  | -2- |  |
| c) | - Why are the arms of a belt pulley gradually tapered from hub to the rim of it. - The mean diameter of a pulley is 300mm and the number of arms is 4. The section of the arms is elliptical with the major axis is equal to twice the minor axis. The tight side and slack side tensions in the belt are 600N and 200N respectively. The permissible tensile stress for the material of the pulley is 15MPa. Determine the crossection of the arms. | (2) (4) |
| Q.No.3. | Answer the following Questions: |  |
| a) a) | Determine the required thickness of the steel bracket at section A-A when loaded as shown in Fig.1 in order to limit the tensile stress to 100MPa. ### OR Find the inside and outside diameter of hollow shaft transmitting 20KW at 200rpm. The ultimate shear stress for the steel may be taken as 360MPa and a factor of safety as 8. The ratio of inside to outside diameter is 0.5. | (3) (3) |
| b) | A countershaft supported between bearings carries two pulleys as shown in Fig.2. Pulley B receives power from motor placed vertically below it. The ratio of belt tensions on pulley B is 2. The pulley C delivers power to machine placed vertically above it. The machine causes the tension of 3KN in tight side of belt and 1KN in the slackside of the belt on pulley C. Determine the diameter of the shaft if the allowable shear stress in the shaft material is limited to 60MPa. Assume combined shock and fatigue factor for Bending and Torsion is KM=2.0 and KT= 1.5. | (9) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cast Iron Bracket is shown in Fig.3. The bracket is fixed to the main body by means of four standard bolts. The tensions in tight side and slack side of the belt are 6KN and 4KN respectively. Neglecting the weight of the pulley, find the size of the bolts if the safe tensile stress is 80MPa. |  |
| b) | A 80mm wide and 12mm thick plate subjected to an axial tension load is welded to a vertical support by a single transverse fillet weld and double parallel fillet weld as shown in fig.4. The maximum tensile and shear stress in the welds are 100MPa and 70MPa respectively. Find the length of each parallel fillet weld if the joint is subjected to static loading. |  |
| c) | - Write a short note on selection of ball bearings from manufacturer’s catalogue. ii)Explain the following terms related to Rolling Contact bearing 1)Dynamic load carrying capacity 2)Bearing life. |  |

………3/-

-3-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | i)What are the requirements of a good coupling? ii) Design a rectangular key for transmitting 1200NM torque to the pulley mounted on it. The permissible stresses for the shaft as well as for key are shear stress of 50MPa and crushing stress of 95MPa. Assume an overload factor of 1.25. | (2) |
| b) | Design a Rigid flange coupling to transmit a torque of 250Nm between two – coaxial shafts. The shaft is made of alloy steel, flanges out of cast iron and bolts out of steel. Four belts are used to couple the flanges. The shafts are keyed to flange hub. The permissible value of stresses are: shear stress on shaft is 100MPa, crushing stress on shaft is 250MPa, shear stress on key are 100MPa, crushing stress on key is 250MPa, shearing stress on C.I is 200MPa, shear stress on bolts in 100MPa. |  |
| c) | A screw jack is to lift a load of 75KN through a height of 350mm. The permissible stresses for screw are t = c = 185MPa (Tension & compression) and I= 95MPa (Shear). The nut is made of phosphor bronze with the permissible stresses are t=100MPa (tension), c = 90MPa (compression) and I= 80MPa (Shear). The permissible bearing pressure between nut and screw is 20N/mm2. The coefficient of friction between nut and screw is 0.12. Design the nut and screw jack. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A helical valve spring is to be designed for an operating load range of approximately 90N to 135N. The deflection of spring for the load range is 7.5mm. Assume spring index of 10. allowable shear stress is 345 MPa. Modulus of rigidity is 78.45 Gpa. Design a suitable helical compression spring. |  |
| b) | A railway wagon weighing 50KN and moving with a speed of 8km/hr has to be stopped by four buffer springs in which the maximum compression allowed is 220mm. The wire diameter of the spring is 25mm. The spring mean diameter is 150mm. Take modulus of rigidity as 84Gpa. Calculate number of turns or coils in each spring. Also find shear stress induced in each spring material and solid length of spring. |  |
| c) | i)explain equalized stresses or nipping in the leaf spring. ii)What is significance of Wahl’s stress factor in the design of helical compression spring? |  |
|  |  | …..4/- |