## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Engineering & Technology

Subject: Applied Mechanics-I(2005) [Old Course]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the concept of rigid body and deformable body. |  |
| b) | Differentiate between vector quantity and scalar quantity. |  |
| c) | Explain in brief the classification of forces. |  |
| d) | Explain ‘Principle of Transmissibility’ with a neat sketch. |  |
| e) | Define ‘Couple’ and state its characteristics. |  |
| f) | Explain with neat sketches the different types of supports. |  |
| g) | Explain the velocity time diagram used in rectilinear motion. |  |
| h) | What is ‘Centre of Gravity’? Why do we need to calculate it? Give two examples. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a jib crane, jib is 5m long and tie is 4m horizontal. The height of post is 3m. If a load of 20KN is suspended at crane head find out the forces produced in jib and tie rods. |  |
| b) | A particle is acted upon by the forces shown in fig.(1). Determine the magnitude and direction of the resultant. |  |
| c) | A particle is acted upon by the following forces: - 200N inclined 30 with East towards North - 250N towards North - 300N towards North West - 350N inclined at 40 with West towards South. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An electric light fixture weighing 15N hangs from a point C by two strings AC and BC, AC is inclined at 60 to the horizontal and BC 45 to the vertical as shown in Fig.(2). Determine the forces in the string AC and BC. |  |
| b) | A horizontal beam 4m long is supported at the ends and carries concentrated loads of 3KN and 5KN at 1m and 3m respectively from the left end. Find the end reactions. |  |
| c) | Two identical rollers each of weight 50N rest on an inclined plane which makes an angle 60 with the vertical wall. Find reactions at all points of contact supports assuming all the surfaces to be smooth. Ref. Fig (3). |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A block weighing 60N resting on a rough horizontal plane can be just moved by a pull of 14N applied at an angle of 40 with horizontal. Find the coefficient of friction. |  |
| b) | A pull of 200N is required just to move a certain body up an incline plane of angle 15, the force acting parallel to the plane. If the angle of inclination of the plane is made 20, the pull required again parallel to the plane, is found to be 230N. Find the weight of the body and the coefficient of friction. |  |
| c) | A uniform ladder of weight 250N and length 5m is placed against a vertical wall in a position where its inclination to the vertical is 30. A man weighing 800N climbs the ladder. At what portion will he induced slipping? The coefficient of friction for both the contact surfaces of the ladder is 0.20. | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A stone was thrown vertically upwards from the ground with a velocity of 98m/sec. After 2 seconds, another stone was thrown vertically upwards from the same place. If both the stones strike the ground at the same time, find the velocity with which the second stone was thrown. Take g= 9.8m/sec2. |  |
| b) | An automobile starting from rest speeds upto 12m/sec with a constant acceleration of 1.2m/sec2, runs at this speed for a time, and finally comes to rest with a acceleration of 1.5m/sec2. If the total distance travelled is 360m, find the total time required. |  |
| c) | Two trains A and B leave the same station on parallel lines. The train A starts with a uniform acceleration of 0.2m/sec2 and attains a speed of 45Kmph which remains constant. The train B leaves 1minute after with a uniform acceleration of 0.4m/sec2 to attain a maximum speed of 72 Kmph. When the train B will overtake the train A. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the C.G of an angle section 100mm x 60mm x 20mm. |  |
| b) | A trapezoidal wall of height 6m has crest width 1.5m and base width 3m. Find its C.G. |  |
| c) | A solid right circular cone of height 12cm and base diameter 4cm is cut by a plane perpendicular to its axis so that its upper half portion is removed. Find the position of the centre of gravity of remaining portion from its base. |  |