## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Control Engineering(4219)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain any three rules of block diagram reduction. |  |
| b) | State Nyquist stability criterion. |  |
| c) | Draw block diagram of open loop and closed loop system. |  |
| d) | With respect to the roots of a characteristic equation explain when the system is stable, unstable and limitedly stable. |  |
| e) | Draw a diagram of rotary type potentiometer and obtain its transfer function. |  |
| f) | State the mathematical equation of an impulse function. Also state its Laplace transform. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the effect of feedback on a control system. |  |
| b) | Give an example of a multivariable control system. Explain the working with the help of block diagram. |  |
| c) | With the help of a block diagram explain the working of any manually controlled closed loop system. |  |
| Q.No.3. | Explain the principle of operation, construction characteristics and application of any two of the following: i) Stepper motor ii) AC iii) Shaft encoder. | (12) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Obtain the mathematical model of a mass, spring, dashpot system. |  |
| b) | For the system shown in figure 4(b) obtain i) Open loop transfer function ii) Closed loop transfer function iii) Characteristic equation. |  |
| c) | For a first order system with transfer function C(s)=1Ts+1, find steady state error for unit step input. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Given the characteristic equation, determine the range of K for the system to be stable. S4+5S3+5S2+4S+K=0 |  |
| b) | What are static error coefficients? Explain the meaning of order, class and type of system. |  |
| c) | Obtain the response of a first order system to a unit step input. Sketch the response curve. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Use Nyquist criterion to determine the stability of the system with open loop transfer function G(s)H(s)=1s(1+6s). |  |
| b) | Draw the bode plot for the system whose transfer function is given by G(s)=(s+1)s(s+10). |  |
| c) | Explain the following time response specifications: i) Delay time ii) Rise time iii) Peak time iv) Settling time. |  |