## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg./FT/A/SB

### Subject: Elements of Electrical & Electronics Engg.(CS302)[Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the power factor of the load. State its minimum & maximum value. |  |
| b) | State the methods used to reduce earth resistance. |  |
| c) | State the colour codes used for single phase & three phase PVC insulated cables. |  |
| d) | Can a d.c. motor be started directly on line? Say yes or no and justify your choice. |  |
| e) | State the three applications of each type of three phase Induction motor. |  |
| f) | Draw the symbols of: 1. PNP Transistor 2. NPN transistor 3. PN diode. Also distinguish between P & N type semi conductors with respect to the quantity of holes & electrons present in each type. |  |
| g) | State the voltage levels at which power is supplied for primary distribution, secondary distribution, power supplied to domestic consumers and commercial consumers. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a single line diagram to indicate the various stages of distribution of power to consumers starting from the point at which it is received. Also indicate at each stage the required voltage level at which power is distributed. |  |
| b) | Define earthing. State the types of earthing electrodes used. Also state the condition under which each is used. State any two necessities of providing earthing. |  |
| c) | State all the parts of three phase PVC insulated power cable. Also state the function of each part. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat diagram explain in brief the working of an electromagnetic relay. |  |
| b) | Draw the symbol of an electromagnetic contactor and give its functions as well as its working. |  |
| c) | Give the constructional features of a rewireable fuse. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the important parts of a single phase transformer. Also give its constructional features and function of each part. |  |
| b) | State the different types of losses we come across in a transformer. Also state the factors affecting these losses. How can they be minimized? |  |
| c) | Explain in brief working principle of a d.c. motor with the help of neat circuit diagram. |  |
| d) | Classify the d.c. motors. Also draw the circuit diagram and give two applications of each type. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief principle of operation of three phase Induction motor. |  |
| b) | Classify the rotors in case of three phase Induction motor. Also draw the diagrams and state their constructional features. |  |
| c) | Draw the symbols of following Gates and give their truth table: 1. NOR 2. EX NOR 3. AND. |  |
| d) | Convert the following number into decimal 1101.0101. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw forward biased & reverse biased characteristic of a PN diode and explain each in brief. |  |
| b) | Draw a neat circuit diagram of CE Amplifier and explain its working. |  |
| c) | Draw a neat circuit diagram of single phase full wave bridge rectifier and explain its working in brief. State its expression for d.c. output voltage and draw its output voltage waveform. |  |

***********