## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg.

Subject: Illumination Engineering(5106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define (i) Light (ii) Radiant efficiency of a luminous body (iii) Natural lighting. |  |
| b) | Define the following terms: (i) Lumen (ii) Brightness (iii) Luminous Intensity. |  |
| c) | State the expression for illumination level at a point on working plane which is exactly below the centre of the linear lighting source. |  |
| d) | Explain lamp lumen method of lighting calculations. |  |
| e) | State any three important features of staircase lighting. |  |
| f) | State the different factors affecting selection of coefficient of utilization. |  |
| g) | Explain ‘diffusion principle’ in the design of street lighting installation. |  |
| h) | Give any three important features of Auditorium lighting. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram, explain direct and indirect lighting system. Also state the applications of both lighting system. |  |
| b) | State and explain different types of reflections we come across in Illumination Engineering. |  |
| c) | Define the following terms: (i) Direct glare (ii) Indirect glare (iii) Discomfort glare (iv) Disability glare. Also state four factors affecting glare. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain Inverse square law of illumination with the help of neat sketch. |  |
| b) | (i) Explain the significance of polar curves. Also explain different types of polar curves. (ii) A lamp having M.S.C.P of 800 is suspended at a height of 10m from ceiling. Determine 1) Total flux 2) Illumination level just below the lamp. |  |
| c) | A 4 feet fluorescent tube is mounted above the front edge of a workbench of 1.5m x 1m size. The mounting height is 1.5m. The transverse intensity of lamp in lower hemisphere is 600cd in all direction. Calculate the illumination level at the centre of the workbench. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the steps involved in the design of interior lighting. |  |
| b) | Give two point of distinction between: (i) Room Index and cavity index (ii) D.L.O.R. and U.L.O.R. (iii) Uniformity ratio and Direct ratio. |  |
|  |  | …..2/- |
|  | -2- |  |
| c) | Estimate the number and wattage of lamps which would be required to illuminate a workshop space 60 x 15 metres by means of lamps mounted 5 m above the working plane. The average illumination required is about 100 lux, CDU = 0.4, luminous efficiency 16 lumens/watt. Assume a s/hm = unity and candle power depreciation of 20%. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat diagram, explain what is i) Carriageway ii)Kerb iii) Outreach iv) Overhang; related to street lighting. |  |
| b) | Explain the classification of luminaires used in street lighting design. |  |
| c) | Classify streets with respect to their average illumination level, uniformity ratio and the type of luminaires preferred and permitted. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is Floodlighting? Explain the classification of Flood light projectors according to their beam spread. |  |
| b) | - State any three important features of sports lighting. - Explain specular reflection principle in the design of street lighting. |  |
| c) | The front of a building 50m x 16 m is illuminated by sixteen 1000 watts lamp arranged so that uniform illumination on the surface is obtained. Assuming luminous efficiency of 17.4 lumens/watt and coefficient of utilization of 0.4, determine the illumination on the surface. |  |

***********