## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-I(4331)/(SH301) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is  compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Length between perpendiculars and moulded breadth |  |
| b) | Archimedes principle |  |
| c) | Body Plan |  |
| d) | Simpson’s 141 rule |  |
| e) | Righting Lever |  |
| f) | Tonnes per cm immersion |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write main features and functions of: (i) Passenger Vessel (ii) Hovercraft. |  |
| b) | The length of a ship at the waterline is 100m, the maximum beam is 15m and the coefficient of water plane area is 0.8. Find the TPC at this draft. |  |
| c) | A ship has a light displacement of 2000 tonnes and light KG 3.7m. She then loads 2500 tonnes of cargo (KG=2.5m) and 300 tonnes of bunkers (KG=3m). Find the new KG. |  |
| Q.No.3. | A ships water plane is 80m long. The half breadths commencing from forward are as follows: 0, 1.52, 3.05, 4.7, 5.1, 5.18, 5.15, 5, 4.42, 2.88 and 0m respectively. The spacing between the first three and last three ordinates is half of that between the other ordinates. Calculate the area of the water plane, and the position of the centre of floatation. | (12) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Explain the procedure for conduct of Inclining Test. (Only for students of Revised scheme) | (6) |
|  | OR |  |
| a) | Explain the types of equilibrium. (Only for students of Old scheme) | (6) |
| b) | A ship of 9000 tonnes displacement has GM=0.61m and KG=5.70m. A d.b. tank containing sea water is being pumped out. Tank dimensions are 15m x 12m x 1.2m. Determine the GM when tank is: (i) Pumped out to half level (ii) Fully pumped out. | (6) |
| Q.No.5. | The second moment of a ship’s water plane area about the centre line is 20,000 m4. The displacement is 7000 tonnes whilst floating in dock water of density 1008 Kg per cu.m. KB=1.9m and KG=3m. Calculate the initial metacentric height. | (12) |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | A ship 100m long arrives in port with drafts 3m F and 4.3m A. TPC=10 tonnes, MCT 1cm=120 tonnes-m. The centre of floatation is 3m aft of amidships. If 80 tonnes of cargo is loaded in a position 24m forward of amidships and 40 tonnes of cargo is discharged from 12m aft of amidships, what are the new drafts. | (12) |
| Q.No.7. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Write in brief about bonjean curves and their application. |  |
| b) | A ship of 12000 tonnes displacement is 150m long at the waterline. The half ordinates of the waterplane at equidistant intervals from forward are: 0, 4, 8.5, 11.6, 12.2, 12.5, 12.5, 11.6, 5.2, 2.4 and 0.3m respectively. Calculate transverse BM. |  |

***********