## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg./P/A/SB

Subject: Strength of Materials(4032)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define stress, stain and Young’s Modulus of elasticity for a member loaded axially in tension. |  |
| b) | Define bulk modulus. State the relationship between Young’s modulus of elasticity and bulk modulus. |  |
| c) | State and explain Parallel axis theorem with an example. |  |
| d) | Define strain energy and proof resilience. |  |
| e) | State bending equation and list out each term involved in the equation. |  |
| f) | What is the limitation of eccentricity for stress to remain wholly compressive for a solid circular section column of diameter ‘d’? |  |
| g) | State Euler’s formula and list each term involved. |  |
| h) | What are the stresses that occur in thin cylinder subjected to an internal pressure? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel bar consisting of three lengths 200mm, 280mm and 180 mm with diameters 30mm, 20mm and 35mm respectively is subjected to an axial pull of 50 KN. Find the stresses in each of the portions and the total extension of the bar. E=2x105N/mm2. |  |
| b) | A tube of aluminium 40mm external diameter and 20mm internal diameter is fitted on to a solid steel rod of 20mm diameter. The composite bar is loaded in compression with 45KN. Find the stresses in aluminium and steel if Es=2x105N/mm2 and Ea=7x104N/mm2. |  |
| c) | A rod 2.5m long at 10ºC is subjected to a raise in temperature by 70ºC. Find the stress in the rod if the expansion of the rod is prevented. Also find free expansion. E=1x105N/mm2, =0.000012 perºC. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The principal stresses at a point in a bar are 200N/mm2 (tensile) and 100N/mm2 (compressive). Determine the resultant stress in magnitude and direction on a plane inclined at 60º to the axis of the major principal stress. Also determine the maximum intensity of shear stress in the material at the point. |  |
| b) | At a certain point in a strained material the principal stresses are 100N/mm2 and 40N/mm2 both tensile. Find the normal, tangential and resultant stresses across a plane through the point at 48º to the major principal plane, using Mohr’s Circle of stress. |  |
|  |  | …….2/- |
|  | -2- |  |
| c) | A steel bar 15mm in diameter is pulled axially by a force of 10KN. If the bar is 250mm long. Calculate the strain energy stored per unit volume of the bar and the total strain energy stored by the bar. Take E=2x105N/mm2. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported beam AB of span 8m is subjected to concentrated loads 6KN, 12KN and 8KN at distances of 1.5m, 4m and 6m from the left support. Draw SFD and BMD. |  |
| b) | A cantilever AB is fixed at A and free at B. It is subjected to point load of 100N at free end and uniformly distributed load of 20N/m over the entire span of 5m. Draw SFD and BMD. |  |
| c) | Find the Moment of Inertia about horizontal centroidal axis of an inverted ‘Tee’ section whose flange is 150mmx20mm, web thickness is 20mm and overall depth is 120mm. Also find the Moment of Inertia of the section about the bottom most fibre. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel ‘I’ section of Moment of Inertia 11641x105mm4 and overall depth 600mm is a section of a beam which is simply supported. The beam carries an uniformly distributed load of 50KN per metre run on entire span of 8m. Calculate the maximum stress produced due to bending. |  |
| b) | A column is 800mmx800mm in section. It carries a point load of 1000KN acting on the centroidal axis at an eccentricity of 90mm from central longitudinal axis. Find the maximum and minimum stresses on the section. |  |
| c) | Find the diameter of a solid shaft to transmit 115KW of power at 225 rpm, if the shear stress is not to exceed 80N/mm2 and the twist shall not exceed 1º in a length of 3.25m. C=8x104N/mm2. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cylindrical drum is 800mm in diameter and has 10mm thickness. If the drum is subjected to an internal pressure of 2.5N/mm2, determine hoop stress, longitudinal stress and circumferential strain. E=2x105N/mm2, Poison’s ratio is 0.25. |  |
| b) | A 1.5 metre long column has a circular cross-section of 50mm diameter. One end of the column is fixed in direction and position and the other end is free. Taking a factor of safety of 3, calculate the safe load using Rankine’s formula. Take fc=560N/mm2 and =1/1600. |  |
| c) | A timber beam 100mm wide, 250mm deep is simply supported over a span of 4m. Find the uniformly distributed load that can be applied on the beam over the whole span so that the deflection at the centre may not exceed 6mm. Take E=1.12x104N/mm2. |  |

***********