## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Power Electronics & Drive Control(5102)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give three points of distinction between UJI and PUT. |  |
| b) | State any three important features of Schottky diode. |  |
| c) | Explain how to recover the trapped energy in an inductive load. |  |
| d) | Explain the circuit used for di/dt protection of a thyristor. |  |
| e) | Draw the circuit diagram and the relevant waveforms of a single phase full wave rectifier (M-2 connection). |  |
| f) | What are cycloconverters? State the different types of cycloconverters. |  |
| g) | Differentiate between Buck regulator and Boost regulator. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the two transistor model of a thyristor. |  |
| b) | What do you understand by commutation of a thyristor? State various methods of commutation and explain any one in detail. |  |
| c) | Explain the various Turn-ON methods of an SCR. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram and relevant waveforms explain single phase Bridge Converter (B2 Connection) circuit with resistive load. |  |
| b) | With a neat diagram and waveforms explain the single phase full wave phase controller with resistive load. |  |
| c) | With the help of circuit diagram and waveforms, explain the working of a single phase cycloconverter, with centre tapped transformer configuration. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of a Boost regulator in detail. Draw the relevant circuit diagram and waveforms. |  |
| b) | With the help of a circuit diagram and waveforms explain the significance of a free wheeling diode. |  |
| c) | Classify Choppers and explain step down Chopper circuit with relevant circuit diagram and waveforms. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give the classification of DC drives. Explain any one DC drive in brief. |  |
| b) | State the methods used in speed control of DC drives. Explain any one of such method of your choice using appropriate thyristorised devices with the help of neat circuit diagram. |  |
| c) | Explain how DC motor control is obtained using chopper controlled drives. |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Stator Voltage Control |  |
| b) | Rotor Voltage Control |  |
| c) | Current Control |  |

***********