## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Resistance & Propulsion(4339)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Tandem rotating propellers |  |
| b) | Reynold’s number |  |
| c) | Sealing in water cooled stern tube |  |
| d) | Skew of propeller |  |
| e) | Propeller slip |  |
| f) | Wind resistance |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | A 6m model of a ship has a wetted surface area of 7m2 and when towed in freshwater at 3 Knots has total resistance of 35N. Calculate the effective power of the ship 120m long at its corresponding speed. h= 1.825 SCF=1.15 Rf= f.s.vn where v is in Knots & f = 0.417 + 0.773L+2.862 | (8) |
| b) | Sketch and describe tandem rotating propellers. | (4) |
| Q.No.3. | Answer the following Questions: |  |
| a) | For model testing explain geometric, kinematic and dynamic similarity existing between ship and model. | (8) |
| b) | Why reduction gears are required in propulsion system? Define efficiency of gear box. | (4) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Explain how the total resistance of the ship can be obtained by conducting model experiment. Explain with equation Froude’s law of similarity. | (8) |
| b) | Explain in brief construction of reduction gear box. | (4) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Sketch and describe oil cooled stern tube. | (8) |
| b) | Sketch and describe a bracket arrangement used to support over hanging propeller shaft. | (4) |
| Q.No.6. | Answer the following Questions: |  |
| a) | When propeller of 4.8m pitch turns at 110 rev/min the apparent slip is found to be -x% and the real slip +1.5x%. If the wake speed is 25% of the ship speed, calculate the ship speed, the apparent slip and real slip. | (8) |
| b) | Define shaft horse power. At what point on the propulsion system it is measured? How shaft horse power and break horse power are related to each other? | (4) ….2/- |
|  | ### -2- |  |
| Q.No.7. | Answer the following Questions: |  |
| a) | A propeller of 5.5m diameter has a pitch ratio of 0.8. When turning at 120 rev/min the wake fraction is found to be 0.32 and the real slip 35%. Calculate the ship speed, speed of advance and apparent slip. | (7) |
| b) | Sketch and describe Schneider propeller. | (5) |

***********