## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg./A

Subject: Thermal Engineering(ME303) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State 2 differences between closed system and open system. Give one example of each. |  |
| b) | Define: i) Work ii) Power iii) Energy. Also state their units. |  |
| c) | State and explain the Kelvin Plank’s statement of second law of thermodynamics. |  |
| d) | State Boyle’s law and Gay Lussac’s law. |  |
| e) | Define boiler mountings and boiler accessories. |  |
| f) | Define mach number. State its significance |  |
| g) | State and explain Dalton’s law of partial pressures. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A domestic food freezer is to be maintained at a temperature of -15ºC. The ambient air temperature is 30ºC. If heat leaks into the freezer at the continuous rate of 1.75KJ/s, find the power required to pump this heat out continuously. |  |
| b) | A vessel of capacity 5m3 contains 20Kg of an ideal gas having a molecular mass of 25. If the temperature of the gas is 15ºC, find its pressure. Take universal gas constant = 8.314KJ/KgK. |  |
| c) | With the help of neat sketch, explain the construction and working of a barrel calorimeter. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A certain quantity of air has a volume of 0.028 m3 at a pressure of 1.25 bar and 25ºC. It is compressed to a volume of 0.0042m3 according to the law pv1.3 = constant. Find the final temperature and work done during compression. |  |
| b) | One Kg of air at a temperature of 40ºC is compressed isothermally from a pressure of 1.5 bar to 6 bar. Determine the heat rejected by the air during the process of compression. For air, take Cp=1.005 KJ/KgK and Cv=0.712 KJ/KgK. |  |
| c) | With the help of neat sketch, explain the construction and working of Babcock and Wilcox boiler. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate the internal energy per Kg of superheated steam at a pressure of 10 bar and a temperature of 300ºC, If the steam is expanded to 1.4 bar and dryness fraction 0.8, find the change in internal energy. |  |
| b) | Steam is supplied to a steam turbine at a pressure of 20 bar and 350C. It is then expanded isentropically to a pressure of 0.08 bar. Determine the Rankine cycle efficiency. |  |
| c) | With the help of a neat schematic, explain the construction and working of Benson boiler. |  |

……..2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With the help of neat sketches, explain the construction and working of any two boiler mountings. |  |
| b) | Explain the nozzle control governing method in steam turbines. |  |
| c) | With the help of a neat sketch, explain the construction of a downflow type surface condenser. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Define the terms: i) Enthalpy ii) Entropy. State their units. |  |
| b) | State any four energy conservation practices one should follow to save energy in their homes. |  |
| c) | One Kg of an ideal gas is heated from 18.3ºC to 93.4ºC. Assuming R=0.264 KJ/KgK and = 1.18 for the gas, find the change in internal energy. |  |
| d) | Define boiler draught. Explain different types of boiler draught. |  |
| e) | State four major differences between impulse turbine and reaction turbine. |  |
| f) | Draw a neat labelled sketch of an induced draught cooling tower. |  |

***********