## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Civil Engg./CT

Subject: Maintenance of Civil Engineering Structures(5003)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the need for maintenance of Civil Engineering structures. |  |
| b) | How an estimate is made for maintenance work? |  |
| c) | How the maintenance of wood work is carried out for buildings? |  |
| d) | Enlist the causes of defects in structural components. |  |
| e) | Explain the maintenance activities required for storage tanks. |  |
| f) | Explain how availability and selection of materials for repair works affects the cost of maintenance. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the importance of design considerations in carrying out repairs work. |  |
| b) | What are the common defects in foundations? Explain the maintenance procedure for repair of these defects. (any three) |  |
| c) | Write a brief note on maintenance of electrical systems in multistoried buildings. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the maintenance works to be carried out in repair of defects in sewage treatment units. |  |
| b) | What are the inspection techniques adopted for water supply system? Explain briefly. |  |
| c) | Write a brief note on fire protection in buildings. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With reference to the maintenance of canals, explain: i) silt removal ii) strengthening of canal banks. |  |
| b) | What are the common defects in WBM roads? How are they rectified? |  |
| c) | Explain the inspection practices for roads. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the repair technique adopted for defects in masonry dams. |  |
| b) | Explain how weed control is carried out in canals. |  |
| c) | Explain briefly the procedure for repair of filters in dams. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Cement grouting |  |
| b) | Leakage detection in pipelines |  |
| c) | Maintenance of irrigation channels |  |
| d) | Pre-monsoon inspection of roads |  |

***********