## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(CE301) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between plane & geodetic survey. |  |
| b) | Explain terms (i) base line (ii) tie line (iii) offset. |  |
| c) | Sketch the conventional symbol for: (i) Embankment (ii) road (iii) river. |  |
| d) | What do you understand by: (i) Change point (ii) Height of instrument (iii) Fore sight in levelling? |  |
| e) | Define bearing. Which are the different systems of recording bearings? |  |
| f) | List various accessories required for plane table survey. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is ranging? Explain direct ranging by eye. Also name instruments used for ranging. |  |
| b) | Explain various methods for determining width of river. |  |
| c) | Explain the factors to be considered in deciding stations of chain survey. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with neat sketch construction of prismatic compass. |  |
| b) | What is local attraction? How is it determined & eliminated? |  |
| c) | Determine values of included angle in closed traverse ABCD conducted in clockwise direction given following FB of respective lines. Line; FB. AB; 40. BC; 70. CD; 210. DA; 280. Apply check. |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Describe HI method of Computing levels. Also sketch level page to record readings. |  |
| b) | Following consecutive readings were taken with level and levelling staff on continuously sloping ground at common interval of 20m. 0.602, 1.234, 1.860, 2.574, 0.238, 0.914, 1.936, 2.872, 0.568, 1.824, 2.722. RL of first point is 192.122. Rule out page of level book. Enter above reading. Calculate RL of points & also the gradient of line joining first and last point. |  |

……..2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What are different sources of error in levelling? How are they eliminated? |  |
| b) | What is B.M? State its type & explain each. |  |
| c) | Explain with neat sketch “Radiation” in plane table survey. |  |
| Q.No.6. | Write short notes on: (any three) | 3 x 4 = 12 |
| a) | Principles of survey |  |
| b) | Open & Close traverse |  |
| c) | Fly levelling |  |
| d) | Arithmetic checks in levelling |  |
| e) | Whole circle bearing system |  |

***********