## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Introduction to Thermodynamics(4334)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State Zeroth law of Thermodynamics. |  |
| b) | An isothermal process of an ideal gas is a hyperbolic process. Justify the statement. |  |
| c) | State the functions of a carburettor. |  |
| d) | Explain why an I.C. engine needs cooling. |  |
| e) | Distinguish clearly between the various qualities of steam. |  |
| f) | With the help of a neat sketch, explain the effect of decreasing the evaporator pressure on the refrigeration system. |  |
| g) | Convert the following readings of pressure to KPa: - 1,50,000 N/m2 ii) 10 bar iii) 0.01 N/mm2. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define specific heat and explain specific heat at constant volume and specific heat at constant pressure. |  |
| b) | Explain the second law of Thermodynamics with reference to i) Kelvin-Planck and ii) Clausius statements. |  |
| c) | 0.12m3 of air at 1.5MPa and 1,500ºC expands adiabatically to pressure of 175 KPa. Find i) the final temperature, and ii) the work done. Take KP=1.0035KJ/KgK and KV=0.7165 KJ/KgK for air. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Prove that the index  in pv = constant for adiabatic expansion of a gas, is the ratio of the specific heat at constant pressure to the specific heat at constant volume. |  |
| b) | In an ideal Otto cycle engine the compression and expansion follow the adiabatic law with the value of  as 1.4. The pressure, temperature and volume at the beginning of the compression are 100 KPa, 40ºC and 0.03 m3 respectively. The pressure at the end of compression is 750 KPa and that at the end of constant volume heat addition is 1,900 KPa. Calculate the temperatures at the end of i) adiabatic compression, ii) constant volume heat addition, and iii) adiabatic expansion. Also find the compression ratio and the air-standard efficiency of the engine. Take KV=0.7165 kJ/Kg K for air. |  |
| c) | In an ideal Diesel cycle, the temperatures at the beginning and end of compression are 32ºC and 615ºC respectively. If the temperature at the end of constant pressure heat addition is, 1,780ºC, determine: (i) the value of the compression ratio, (ii) the percentage of the working stroke at which cut-off takes place, and (iii) the ideal thermal efficiency. Assume  =1.4 and KP=0.997 KJ/Kg K for air. |  |

……….2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With the help of neat sketches, explain the working of a two- stroke cycle diesel engine |  |
| b) | With the help of a neat sketch, explain the pressure-feed lubrication system of an I.C. engine. |  |
| c) | The following data refer to a test on a four–stroke cycle oil engine: Number of cylinders = 4; cylinder bore = 9cm; piston stroke = 13 cm; speed = 2,100 r.p.m; brake torque = 167 N.m; fuel oil consumption = 13.5 litres/hr.; calorific value of fuel oil = 44,000 KJ/Kg; specific gravity of fuel oil = 0.82. Calculate: (i) the brake mean effective pressure (b.m.e.p.) (ii) the brake thermal efficiency, and (iii) the brake specific fuel consumption in litre per KW/hr. |  |
| d) | What is meant by saturation temperature? What is the effect of increase in pressure on saturation temperature, enthalpy of saturated water, enthalpy of evaporation and enthalpy of dry saturated steam? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the condition of steam in each of the following cases: - At a pressure of 10 bar (1 MPa) and temperature 200ºC. - At a pressure of 8 bar and volume 0.22m3/Kg, and - At a pressure of 12 bar, if 2,688 KJ/Kg are required to produce it from water at 0ºC. |  |
| b) | The internal energy of 1 kg of steam at 10 bar is 2,400 KJ. Calculate the dryness fraction of this steam. Also find the increase in internal energy when the above steam is superheated at constant pressure to a temperature at 250ºC. Take KP of superheated steam as 2.1 KJ/Kg K. |  |
| c) | Define the following terms as applied to heat transfer by radiation: - Emissivity - Absorptivity and - Reflectivity. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify various heat exchangers and draw diagrams/graphs showing temperature distribution along their lengths. |  |
| b) | Discuss the classification of various refrigeration applications. |  |
| c) | With the help of a neat sketch, explain the open air refrigeration system using the Bell-boleman cycle. |  |

***********