## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Civil Engg./CT

Subject: Irrigation Engg.(4017)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Show the classification/types of irrigation systems. |  |
| b) | Define G.C.A, C.C.A and commanded area. |  |
| c) | What are the functions of storage reservoirs? Write pointwise. |  |
| d) | Draw a neat sketch of theoretical profile of a gravity dam and show all forces acting on it. |  |
| e) | State the three classified failures of earth dams and subtypes of failure in each class. |  |
| f) | What are the various investigations before selecting a suitable site for irrigation project? |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain with a sketch the Symons rain gauge. |  |
| b) | Write any 3 methods to compute run off. |  |
| c) | What are the three methods of calculating average rainfall? Explain any one. |  |
| d) | Explain Duty and Delta. State their units of measurement. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | An irrigation canal has a G.C.A of 90,000 ha out of which 80% is culturable irrigable land. Intensity of irrigation for Kharif season is 30% & for rabi is 60%. Find the discharge required at the head of the canal if the duty at its head is 800ha/cumec for Kharif and 1800 ha/cumec for rabi. |  |
| b) | Define outlet factor, cumec day. Write any 6 points with brief explanation on how to improve duty of water. |  |
| c) | What are the reservoir losses? Enlist any four points on how to minimize losses in reservoir. |  |
| d) | Write any eight points on factors to be considered for selection of site for a reservoir. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the control levels for a medium sized reservoir from the following data: Effective storage required for crops: 3000 ha-m Tank losses: 20% of effective storage Dead storage: 10% of gross storage |  |
| Water level in m; 200; 225; 250; 255; 260; 265; 275; 285; 290; 295. Capacity in Mm3; 2.8; 11.2; 23.4; 31.6; 34.5; 41.0; 51.0; 62.0; 71.5; 77.0. |
| b) | - What is a gravity dam? What are the forces causing stability and the forces causing instability/failure of a gravity dam? - Write short notes on pickup weir and their functions. |  |
| c) | - Write notes of galleries in gravity dam with sketches. - Write short notes with sketches on joints in gravity dam. |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.5. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Mention any types of spillways and explain in brief each. |  |
| b) | Draw a neat layout plan of a bandhara irrigation system and explain advantages of this system of irrigation. |  |
| c) | What are the advantages of canal lining? Write any 6 points. |  |
| d) | Explain in brief the two methods of construction of earth dams. |  |
| e) | Write any 6 important advantages of irrigation. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Weirs and barrages |  |
| b) | Drainage in earth dams |  |
| c) | Level crossing-inlets and outlets in cross drainage works |  |
| d) | Requirements of an ideal site for construction of a dam reservoir |  |
| e) | Vertical gates and radial gates in spillways |  |
| f) | Yield of wells and method to estimate it |  |

***********