## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Machine Technology(4217)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State and explain the working principle of impulse turbine. |  |
| b) | Explain the effect of intercooling in a multistage reciprocating compressor. |  |
| c) | What is a gear train? What are its main types? |  |
| d) | What are advantages of using a condenser in a steam power plant? |  |
| e) | Differentiate between rotary and reciprocating compressor. |  |
| f) | What is priming? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define: (i) Kinematic link (ii) Kinematic pair (iii) Kinematic chain. |  |
| b) | Explain with a neat sketch Whitworth’s quick return mechanism. |  |
| c) | Two shafts with centre to centre distance between them as 5m are having two pulleys with radii equal to 250mm and 350mm respectively. Find the length of the belt required for an open belt drive. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch construction and working of a platform scale. |  |
| b) | What is the function of a coupling? With a neat sketch explain construction of any flexible coupling known to you. |  |
| c) | With a neat sketch explain construction and working of a Lancashire boiler. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the functions of a fusible plug. With a neat sketch explain construction and working of a fusible plug. |  |
| b) | Describe with a neat sketch the working of a two–pass surface condenser. |  |
| c) | Explain with a neat sketch a low level jet condenser. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with a neat sketch Pressure – velocity compounded turbine. |  |
| b) | What are boiler accessories? Explain with a sketch an economiser. |  |
| c) | With a neat sketch explain construction and working of a Vane type of compressor. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with a neat sketch the working of a reciprocating pump. |  |
| b) | State the working principle of water–ring vacuum pump. Explain its construction with a neat sketch. |  |
| c) | Explain with a neat sketch the second inversion of double slider crank mechanism. |  |

***********