## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-I(4022)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define term (i) Flitched beam (ii) Transformed section. |  |
| b) | Derive the expression for limit of eccentricity for a circular column carrying eccentric load. |  |
| c) | Determine the maximum shear force of simply supported girder of span 20m at the centre if a rolling load of 25KN passes through it. |  |
| d) | Draw influence line diagram for reaction in case of simply supported beam for bending moment with analytical proof. |  |
| e) | Explain significance of Mohr’s circle in analysis of complex stress system. |  |
| f) | State advantages and disadvantages of indeterminate structures. |  |
| g) | What is the purpose of ILD and rolling loads? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A timber beam 150 mm wide and 250 mm deep is reinforced by a steel plate of 12mm thick and 200mm deep connected to both sides of timber beam, arranged symmetrically. If stress in steel is 120 N/mm2, find corresponding stress in steel and also the moment of resistance of beam. Take Es = 18 Ew. |  |
| b) | A rectangular wooden beam 100mm wide and 300mm deep is strengthened by connecting a steel plate of 100mm wide and 12mm thick both at top and bottom of wooden beam. If the beam is subjected to a bending moment of 2KNm, determine the stresses produced in steel plate and wood. Assume Es = 15Ew. |  |
| c) | Find out the degree of determinancy of: - A fixed beam subjected to vertical loads only, what would be the state of beam if one support beam is a simply supported. - Portal frame with one support hinged and other fixed. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A short cast–Iron column of hollow circular section has a projecting bracket carrying a load of 60KN, the load line is off the axis of column by 300mm. The size of column is 350mm outside diameter and 300mm internal diameter. Calculate the maximum and minimum intensities of stresses in the column section. |  |
| b) | A rectangular pier of 1500mm x 3000mm (deep) is subjected to an eccentric load of 700 KN. Eccentricity of load above XX-axis is 300mm and from YY-axis is 250mm. The load is placed in Ist Quadrant. Calculate maximum and minimum stress intensities in the section. |  |
|  |  | …….2/- |

- 2 -

| c) | i) Calculate the BM at the centre of a 10m span beam for simply supported and having udl of 6KN/m on the right half span, used ILD. ii) What would be the BM at a section 4m from the left support for the above beam with same loading? |  |
| --- | --- | --- |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported beam of 10m span is carrying point loads of 10KN and 15KN at a distance of 2m & 4m from left end respectively. Also a udl of 10KN/m for a length of 4m starting form right end. Using ILD, determine the reactions at supports and also the SF and BM at a section of 4m from left end. |  |
| b) | A simply supported girder has a span of 12m. A 100 KN wheel load moves from one end to the other end on the girder. Find the maximum BM which can occur at a section 4m from the left end. Also find maximum S.F. at this section. |  |
| c) | A live load of 50KN/m for a length of 10m moves on a span of simply supported girder of 16m span. Find the maximum bending moment which can occur at 7m from left end. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The principal stresses at a point in a strained material are 100 N/mm2 (tensile) and 80 N/mm2 (compressive). Find Normal, tangential and resultant stresses on a plane inclined at 60 to the plane of tensile stress acting on x-axis. |  |
| b) | A material is subjected to normal stresses of 80N/mm2 tensile on x-axis and 50N/mm2 tensile on Y-axis. Also a shear stress of 40 N/mm2 across both planes carrying normal stresses. By Mohr’s circle method determine the principal stresses and the maximum shear stress both in magnitude and direction. |  |
| c) | At a point in a strained material the major and minor principal stresses are 120 N/mm2 (tensile) and 60 N/mm2 (Compressive) respectively. Find the planes in which the stress is wholly shear. Find magnitude of shear stress. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A solid shaft is subjected to a torque of 6000Nm. Find the necessary shaft diameter if shear stress is not to exceed 70N/mm2. Also find the power generated by the shaft if it rotates at a speed of 120 rpm. |  |
| b) | A hollow circular shaft of 250mm outside diameter and 190mm internal diameter transmits 50 KW power at 120rpm. Determine the shear stress produced and angle of twist occurred if the length of shaft is 1.2m. Take C = 0.8 x 105 N/mm2. |  |
| c) | A solid circular shaft of 130mm diameter is subjected to a bending moment of 7 x 106 N-mm and a torque of 4 x 106 N-mm. Determine the principal stresses both in magnitude and direction. |  |

**********