## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics Engg./E/EI/COMP/EC/MED/E&EE

Subject: Digital Electronics-I(4134) [Old Course]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Which of the following systems are analog and which are digital? Why? - Clinical thermometer - Ordinary electric switch - Pressure gauge |  |
| b) | State De Morgan’s theorems. Briefly explain their significance with the help of basic logic gates. |  |
| c) | Explain the problem associated with J=K=l in the ordinary single J-K flip flop. Draw truth table. |  |
| d) | Briefly explain the concept of a Modulo-N counter giving example. |  |
| e) | Explain how a shift register can be used for division and multiplication operation. |  |
| f) | Explain why CMOS family is suitable for battery operated systems as compared to TTL family. |  |
| g) | Briefly explain the concept of Tristate logic. Draw the diagram. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Convert the following numbers to hexadecimal and then to binary: i) 425 ii) 156 iii) 16.86 |  |
| b) | Prove the following: i) A+A.B+A.B=A+B ii) A.B+A.B+A.B=A+B iii) ABC+ABC+ABC+ABC=AB+BC+CA |  |
| c) | For the expression Y=AB+AB - Obtain the truth table - Name the operation performed - Realize this operation using AND, OR, NOT gates - Realize this operation using only NAND gate. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit diagram of a clocked RS FF using NAND gates. Include the PRESET and CLEAR inputs. Explain the operation. Also draw the truth table. |  |
| b) | Draw neat circuit diagrams and explain how the JK FF can be used to avoid the forbidden condition in a RS FF. Also draw the required truth table. |  |
| c) | Explain the truth table of - D FF - T FF Explain how the above can be realized using J-K FF or R-S FF. |  |
|  |  | ……..2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat circuit diagram explain the working of: - PISO shift register - SISO shift register. |  |
| b) | Draw a neat circuit diagram and explain the working of a 3-bit binary ripple counter. |  |
| c) | Explain with the help of an example, the step by step procedure for designing a synchronous counter using JK FFs. Draw relevant diagram. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following terms for a logic family. - Fan out - Propagation delay - Figure of merit - Power dissipation - Low level input voltage - Noise margin |  |
| b) | Draw a neat circuit diagram and explain the working of a basic TTL NAND gate with totem pole output. |  |
| c) | With the help of a neat schematic diagram explain the operation of a CMOS NOR gate. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Gray code |  |
| b) | Ex-NOR gate |  |
| c) | Full adder |  |
| d) | Ring counter |  |
| e) | ECL logic family |  |

***********