## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Welding Technology-II(4240)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the accessories used in Gas Tungsten Arc Welding (GTAW)? State at least one function of each. |  |
| b) | Explain in brief the concept of self controlled arc in Gas Metal Arc Welding (GMAW). |  |
| c) | Compare Flux Cored Arc Welding (FCAW) with Submerged Arc Welding (SAW). |  |
| d) | What are the difficulties encountered in underwater welding? |  |
| e) | What is carbon equivalent? Explain in brief the importance of carbon equivalent in welding. |  |
| f) | Explain in brief the post welding operations in repair and maintenance welding. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the principle of operation of Gas Tungsten Arc Welding (GTAW). |  |
| b) | What are the safety precautions to be taken while carrying out Gas Tungsten Arc Welding (GTAW)? |  |
| c) | With a neat sketch explain the wire feed mechanism used in Gas Metal Arc Welding (GMAW). |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the methods used for starting of arc in Submerged Arc Welding (SAW). |  |
| b) | What are the probable causes of weld porosity in Submerged Arc Welding? How it is prevented? |  |
| c) | Sketch the various types of weld joints used in Electro Slag Welding. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between Electro Slag Welding and Electro Gas Welding. |  |
| b) | With a neat sketch explain the principle of operation of Plasma Arc Welding. |  |
| c) | Explain Resistance Spot Welding cycle. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the principle of operation of Resistance Butt Welding. |  |
| b) | Explain the welding sequence used to control distortion in welding. |  |
| c) | Explain the distortion control techniques used in Arc cutting. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Weldability |  |
| b) | Principles governing design of good welding Jigs and fixtures |  |
| c) | Types of Jigs & Fixtures |  |
| d) | Hard facing of worn-out surfaces |  |
| e) | Effects of operating variables in Flux Cored Arc Welding |  |

***********