## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg.

Subject: Energy Management(5108)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define: (i) Demand factor (ii) Load factor. |  |
| b) | What is energy conservation? List its benefits to the industries. |  |
| c) | What is preliminary audit? State its importance. |  |
| d) | What is colour-rendering index? Briefly explain its importance in selection of lamps. |  |
| e) | Briefly explain how localised switching will improve efficiency of lighting system. |  |
| f) | Briefly explain concept of VFD. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you mean by load side energy management? Briefly explain the techniques used in load side energy management. |  |
| b) | State and briefly explain any six features of Energy conservation Act 2001. |  |
| c) | What is detailed energy audit? Explain methodology employed for detailed energy audit. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you mean by tariff? State its objectives. Briefly explain flat rate tariff. |  |
| b) | Briefly explain different types of power factor tariff. |  |
| c) | Calculate annual bill of a consumer whose maximum demand is 100 KW, pf=0.8 lagging and load factor is 60%. The tariff is Rs. 150 per KVA of maximum demand and Rs. 3 per unit consumed. What will be annual saving in bill if p.f is raised to 0.95 lagging? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain need of electrical load management? Briefly explain how better load management can help to minimize peak demand. |  |
| b) | What is power factor? State advantages of improving p.f. How it is done? |  |
| c) | A factory takes a load of 200 KW at 0.85 p.f. lagging for 2500 hour per annum. The tariff is Rs. 400 per KVA plus Rs. 2/- per KWh consumed. If the p.f is improved to 0.9 lagging by means of capacitor costing Rs. 600 per KVAR and having a power loss of 100 W per KVA, calculate annual saving effected by their use. Allow 10% per annum for interest and depreciation. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain fixed and variable losses in motor. Explain in brief how each of these losses can be reduced in energy efficient motors. | …….2/- |
|  | -2- |  |
| b) | Explain the procedure for assessment of lighting systems. |  |
| c) | Briefly explain the terms: i) Pump efficiency ii) Hydraulic power. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how maximum demand controller works. |  |
| b) | Briefly explain importance of following in energy efficient lighting: i) Occupancy sensors ii) Daylight linked control. |  |
| c) | What is role of ballast in tube light circuit? Explain how electronic ballast saves energy and improves light output. |  |

***********