## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Applied Electronics(4213)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Sketch the two port model of a transistor and list the various hybrid parameters. |  |
| b) | Compare transformer coupled amplifier with direct coupled amplifier. |  |
| c) | Distinguish between the different types of power amplifiers in terms of efficiency and distortion. |  |
| d) | State and briefly explain the Barkhausen’s criteria for sustained oscillations. |  |
| e) | What is meant by amplitude modulation? Sketch the frequency response of AM wave. |  |
| f) | Comment on features of differential amplifier. |  |
| g) | List the characteristics of an ideal opamp. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Briefly explain the method to obtain hybrid parameters from input characteristics of transistor. |  |
| b) | Explain the frequency response of transformer coupled amplifier. |  |
| c) | Justify the need of power amplifier in communication systems. |  |
| d) | Compare AM with FM. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a circuit diagram explain the operation of a voltage series feedback amplifier. |  |
| b) | When negative voltage feedback is applied to an amplifier of gain 100, the overall voltage gain falls to 50. Calculate the feedback fraction value. If this fraction is maintained, calculate the amplifier gain if overall stage gain is to be 75. |  |
| c) | Explain the working of a transistorized RC phase shift oscillator with a circuit diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the term ‘Modulation’. Justify its need in communication. Also comment on phase modulation. |  |
| b) | What is meant by frequency modulation? Explain the generation of FM using basic reactance modulator. |  |
| c) | Draw and explain the block diagram of an operational amplifier. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain V to I and I to V converters using opamp with relevant diagrams. |  |
| b) | Write a short note on first order low pass butterworth filter with opamp. |  |
| c) | Explain the circuit of opamp as an integrator. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Explain the concept of virtual ground in opamps. |  |
| b) | Comment on advantages and disadvantages of RC coupled amplifier. |  |
| c) | Write a short note on hybrid equivalent circuit of CE amplifier. |  |
| d) | Explain the meaning of CMRR in detail. |  |
| e) | Draw and explain the circuit of class B push pull power amplifier. |  |

***********