## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Elementary Ship Design(5331)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Attempt any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Design feature of an oil tanker |  |
| b) | Specifications and contract |  |
| c) | Deadweight carrier versus volume carrier |  |
| d) | Freeboard |  |
| e) | Basis vessel in ship design |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain typical owner’s requirement for construction of a merchant vessel. |  |
| b) | What are the various components of the light ship weight? How are they determined? |  |
| Q.No.3. | Determine the dimension of a bulk carrier of 16000 tonnes DWT at a maximum draft of 8.1m with a service speed of 15 Knots. Given Cd=0.67 B=L9+6 CB=1.00−0.23VL | (12) |
|  | Where Cd= dead weight coefficient |  |
|  | CB= Block coefficient |  |
|  | V= service speed in knots |  |
|  | L= Length in metres |  |
|  | B=Breadth in metres |  |
| Q.No.4. | Explain how the section area curve of a basis ship is modified for a new position of LCG of the vessel and for a new prismatic coefficient. | (12) |
| Q.No.5. | How is the minimum freeboard requirement of a vessel determined? Explain the effect of freeboard on the design of vessel. | (12) |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | What is a general arrangement plan? Explain requirement of crew and passenger accommodation. |  |
| b) | Draw the typical general arrangement plan of a bulk carrier. |  |
| Q.No.7. | Preliminary design of a cargo ship led to the following dimensions: Length =120m, Breadth=18m, Draft=5.6m and CB=0.66. If the draft is increased to 7m, estimate the likely change in the CB and increase in displacement. | (12) |

***********