## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical & Electronics Engg.

### Subject: Power Generation, Transmission & Distribution(5091)[Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the significance of higher values of diversity factor and demand factor? |  |
| b) | Discuss why pulverized coal is used in steam power plants. |  |
| c) | Why bundled conductors are used for overhead transmission lines? |  |
| d) | Explain why suspension type insulators are used for EHV lines. |  |
| e) | Define the terms i) feeder ii) distributor iii) service mains. |  |
| f) | Explain in brief why single pole switch is to be provided in live wire and not in neutral wire. |  |
| g) | Why is earthing essential? What are the factors affecting earth resistance? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A proposed station has following daily load cycle. Time (Hrs) 6-8 8-12 12-16 16-19 19-24 24-6 Load (MW) 30 40 60 80 40 30 Draw the load curve and select suitable generator units. Prepare operation schedule for machines selected. |  |
| b) | Describe working of nuclear power plant with the help of a neat diagram. |  |
| c) | State the methods used for starting of a D.G. set and explain each in brief. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the causes of failure of insulators and methods to overcome them. |  |
| b) | What are the factors which affect corona? What are the methods of reducing corona losses? |  |
| c) | Compare single circuit and double circuit design of towers as support structures. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State any four advantages and two demerits of EHV transmission system. |  |
| b) | What is a strain insulator? Where is it used in the transmission line? Draw a sketch to show its location. |  |
| c) | Derive and compare the volume of conductor material required in AC three phase three wire system and AC three phase four wire distribution system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following in brief: i) radial system ii) ring main system iii) interconnected system. |  |
| b) | Write a short note on voltage and permissible voltage drop in various parts of distribution system for urban and semi urban areas. | …..2/- |
|  | -2- |  |
| c) | A room of size 4m x 4m x 3.5m requires 4 tube light (40w) and 3 fans (60w). - Draw a simple layout showing location of fans, tubes and switch board. - For PVC casing capping wiring, estimate total quantity of material required. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss IE rules for domestic wiring system. |  |
| b) | - With a neat diagram explain godown wiring. - State any three I.E. rules for earthing. |  |
| c) | With a neat diagram explain plate earthing. |  |

***********