## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mining Engg.

Subject: Mining Technology(MN302) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the different purposes of drilling? |  |
| b) | Name the types of drill bits used for blast holes. |  |
| c) | Give the characteristics of a booster explosive. |  |
| d) | What are the different safety issues associated with blasting? |  |
| e) | How is safari support installed in a gallery? |  |
| f) | “Circular shafts are more popular compared to other shape of shafts” – justify. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the characteristics of slurry explosive. |  |
| b) | Classify the explosives based on Indian Explosive Rules and also based on composition of explosives. |  |
| c) | - With a neat sketch explain the construction of a plain detonator. - Write short note on “Shock Tube”. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the types of controlled blasting? Give a brief note on each. |  |
| b) | - Differentiate between direct and inverse initiation. - What is solid blasting? Give its advantages. |  |
| c) | Using emperical formulas give a blast design to extract 3000T of coal by using a drilling machine of 100mm hole dia. Also calculate the powder factor, given the density of explosive to be 1.15gm/cc. Assume other data if required. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the principle of roof bolting? Explain any one type of roof bolt. |  |
| b) | - Explain the principle of hydraulic prop. - Give the classification of supports. |  |
| c) | Enumerate the factors that govern the selection of drilling method. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Describe the freezing method of shaft sinking with a neat sketch. | (8) |
| b) | Mention the types of raising methods. Give a brief note on two- compartment method. | (4) |
|  | ### OR |  |
| b) | Explain with a neat sketch temporary lining of a shaft. | (4) |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the construction of a wagon drill. |  |
| b) | What are the safety measures incorporated in a magazine? |  |
| c) | Write short notes on systematic support rules. |  |
| d) | How is ventilation and lighting done in a sinking shaft? |  |

***********